A

Detailed

Structural Analysis and Design Report

On

RCC FRAME STRUCTURE RESIDENTIAL BUILDING

Submitted to

SURYABINAYAK MUNICIPALITY-9

Owner:  Mr. Ram Pati

TO WHOM IT MAY CONCERN

This report comprises the summary of the structural design of the RCC frame building for Suryabinayak Municipality-9. This report consists of the design procedures adopted, the assumptions made, the inputs made in the design and the design output. During the design, it is assumed that the client will completely follow the architectural as well as the structural design. It is also assumed that the construction will be supervised by a professional engineer.

The designer will not be responsible if any alterations to the structural system is made by the client or the contractor without the prior written permission from the designer, or the alterations to the non-structural system is made such that the weight of each individual floor or the weight of the whole building is altered by more than 10% of the design weight of each floor and the total weight.

The design calculations and the derivations are limited to only a minimum to let the concerned people know the methodology adopted. However, the calculations may be provided to the client or the concerned authorities when needed, upon request.

- TABLE OF CONTENTS

1.0	INTRODUCTION	1

1.1	General	1

1.2	Assumptions	1

1.3	Salient Features	2

1.3.1	Project Information	2

1.3.2	Building Features	2

1.3.3	Material Specification:	2

1.3.4	Site Condition	3

2.0	DESIGN APPROACH AND METHODOLOGY	4

2.1	General	4

2.2	Codes and Standard used	5

2.3	Software Used	6

3.0	ANALYSIS	6

3.1	Material properties	6

3.1.1	Concrete	6

3.1.2	Reinforcement Steel	6

3.2	Section Properties	7

3.3	Loadings	7

3.3.1	Load Cases	7

3.3.2	Load Combination	8

4.0	MODELLING OF THE STRUCTURE	9

4.1	Three Dimensional views of Model	9

4.2	Joint ID	10

4.3	Section Property	11

4.3.1	Beam and Columns	11

4.3.2	Slab	14

4.4	Application of loads	14

4.4.1	Dead Loads	15

4.4.2	Live Load	17

4.4.3	Floor Finish	18

4.5	Seismic Loading	19

4.5.1	Table 6: Mass Source for Seismic Load	19

5.0	ANALYSIS OUTPUT	20

5.1	Seismic Coefficient	20

5.2	Acceptability Criteria:	21

5.2.1	Inter-story drift:	21

5.2.2	Storey Displacement:	23

5.3	Modal time period and mass participation	24

6.0	ANALYSIS OF THE STRUCTURE	25

6.1	Axial Force Diagram	26

6.2	Shear Force Diagram	26

6.3	Bending Moment Diagram	27

6.4	Reinforcement from Etabs	28

6.4.1	Columns	28

6.4.2	Beams	33

7.0	DESIGN OF STRUCTURAL ELEMENTS	37

7.1	Design of columns	37

7.2	Design of beams	40

7.3	Design of Slab	43

7.4	Design of Staircase	47

7.5	Design of Footing	51

8.0	CONCLUSIONS AND RECOMMENDATION	60

9.0	REFERENCES	62

# INTRODUCTION

## General

This report summarizes the structural analysis and design of RC framed building. It has planned to utilize the building for residential purpose. The structural analysis in this building considers three floors only. The aim of design is the achievement of an acceptable probability that structures being designed will perform satisfactorily during their intended life. With an appropriate degree of safety, they should sustain all the loads and deformations of normal construction and use and have adequate durability and adequate resistance to the effects of misuse and fire. Structural Analysis of the concerned building has been done in details with analysis and designs software.

The analysis and design has been based on the prevailing codes that are in practice in Nepal, the National Building code of Nepal (NBC105:2025) and the IS code at places if required. This report consists of the design procedures adopted, the assumptions made, the inputs made in the design and the design output.

## Assumptions

The following assumptions are taken into consideration in the seismic resistant analysis and design of structures:

Adequate supervision and quality systems are provided during execution of the works.

Construction is carried out by personnel having the appropriate skill and experience.

Construction materials and products confirm to the pertinent codes and specifications.

The structure is adequately maintained.

The structure is used in accordance with the design brief.

An earthquake is not likely to occur simultaneously with maximum flood, wind, waves or tides.

Resonance as visualized under steady state sinusoidal excitation will not occur, as the small duration of earthquake is not enough to build up resonance amplitudes. Subsoil does not considerably settle or slide due to earthquake at the site of structure.

This report is divided into three chapters. Chapter one begins with introductions and general description Chapter two comprises the numerical modeling, analysis and design; Chapter three presents the summary and recommendations.

## Salient Features

### Project Information

| Type of building | : | Residential Building |
| --- | --- | --- |
| Location |  | Suryabinayak Municipality -9, Nangkhel , Bhaktapur |
| Plinth Area | : | 1135.61 Square ft. |
| No. of Storey | : | 4 & half Storey |
| Total Floor Area | : | 5654.61 Square ft. |

### Building Features

The building has some special features which are listed below:

| Type of Building | : | Special RC Moment Resisting Frame Structure |
| --- | --- | --- |
| Shape | : | Rectangular Shape |
| Plinth level | : | As per architectural drawing |
| Roof floor Type | : | Accessible, Terrace |
| Walls | : | Brick walls |
| Footing Type | : | Strap Footing, Isolated Footing |
| Depth of foundation | : | Min'n 5 ft. below ground level or as per site condition |
| Dimension | : | 51’-11” x 27’-9” |
| Storey Height | : | 9’-6” (remaining) 10'-6" (Ground and 1st floor) |
| Total Height | : | 52’-6” (from Plinth Level) |
| No of Storey | : | Ground + Three & half Storey |

### Material Specification:

Considering Architectural, Economic and strength demands reinforced cement concrete (RCC) is used as the major structural material. The selected material also confirms the availability and ease in construction.

Concrete Grade		:	M25 for all Column, Beam, Slab, Foundation etc.

Steel Grade                             :	Fe500 for Longitudinal Reinforcement and Shear/Tie Reinforcement

Detailed Properties of Material

A. Concrete

M25 = 25 MPa

Characteristic value of cube compressive strength: Cube 150 x 150 x150 mm (5%-quartile = no more than 5% of cubes tested at 28 days are expected to fail)

Fck, cube 	= 	25 MPa

Fcm		=	 26.6MPa (Mean compressive strength:fcm = fck + 1.65x4.0 N/mm2)

fctm		= 	2.6 MPa (Mean value of concrete tensile strength)

τck		=	1.11 Mpa (Characteristic value of concrete shear strength: = 0.25 √fck)

Ecm		=	 22,360.7MPa 	Mean characteristic modulus of elasticity: 5000√fck

Density 	= 	25 KN/m3

B. Reinforcement

Steel grade	= 	Fe500

Type		= 	Deformed bar

fy		= 	500 MPa Characteristic proof strength at 0.2% yield

εuk		>=	 14.5% Ultimate Elongation

E 		= 	200,000 MPa Elastic Modulus (Young’s Modulus of Elasticity)

Density 	= 	7850 Kg/m3

### Site Condition

Type of soil  	:Type D, Soft Soil as per NBC 105:2025

Safe bearing pressure : 120 kN/m2 (Assumed)

Seismic Zone Factor 	: 1.0 (for Kathmandu Valley) as per NBC 105:2025

# DESIGN APPROACH AND METHODOLOGY

## General

After completion of Architectural design, the layout of columns and beams are done without affecting the Architectural functions of building so far. Structure is modeled using finite element method. A three-dimensional beam element having 12 DOF with 6 DOFs at each node were used for modeling beams and columns in the building, while 24DOFs shell element with 6 DOFs at each node were used to model slab wall.

The structure is analyzed by the linear elastic theory to calculate internal actions produced by anticipated design loads. The analysis is carried out using state of art three dimensional structural analysis programs like ETABS 2022 V 22.7.0. The design loads considered as per the relevant codes of practice comprise dead load due to permanent structures, live load due to occupancy of the structure and seismic load due to anticipated earthquake possible at the proposed location. A number of load combinations are considered to obtain the maximum values of design stresses.

Following considerations is made during modeling, analysis and design.

- The structures are Special Reinforced Concrete Moment Resisting Frame (SMRF) type. Beams and columns are considered as the structural load resisting elements. Although non-structural components like wall plaster, infill walls, floor finishing, etc. has comes effects on structural performance, they are considered only as loading.

- For structural elements, M25 grade concrete are used.

- Centre-line dimensions are followed for modeling, analysis and design.

- Floor slabs are assumed to be rigid in their own plane. The slab action has been modeled by rigid floor diaphragms. Slabs are also considered in modeling. Slab is modeled as shell element.

- Beam and columns are modeled as frame elements.

- The main beams rest centrally on columns to avoid local eccentricity.

- Foundation is assumed to be fully rigid at the plinth level.

- The beam-column joint is not modeled in detail.

- Preliminary sizes of structural components are assumed by experience.

- Seismic loads were considered acting in the horizontal direction (along either of the two orthogonal directions of building) and not along the vertical direction, since it is not considered to be significant for design of structural members suitable load combinations as suggested by NBC 105: 2025 are used.

Figure 3: 3D-Model of Building

## Codes and Standard used

For the structural analysis and design, the following codes and standard are followed:

- IS 456- 2000  Code of practice for plain and reinforced concrete

- IS 875-1987  Code of practice for design loads (other than earthquake) for  buildings and structures

- IS 1893-2002  Criteria for Earthquake Resistant Design of Structures,

- IS 13920-1993  Code of practice for ductile detailing of reinforced concrete structures subjected to seismic forces

- NBC 105:2025 Nepal Building Code

## Software Used

The following software is used for the structural analysis and design.

ETABS Ultimate C 22.7.0 For analysis and design of the structures

For structural modeling of the present building, ETABS software was used. ETABS is a special purpose finite element analysis and design program developed specifically for building systems. With ETABS, models are defined logically floor-by-floor, column-by-column, bay-by-bay and wall-by-wall and not as a stream of non-descript nodes and elements as in general purpose programs. The software has very powerful numerical methods, design procedures and international design codes, all working from a single comprehensive database. At its core, it utilizes the same analysis engine as used by SAP2000.

Among others, ETABS can do model generation, seismic and wind load generation, finite element-based linear and non-linear static and dynamic analysis, concrete frame design (column and beam) and shear wall design.

# ANALYSIS

## Material properties

### Concrete

Structural elements are designed for M25 grade as specified.

Modulus of Elasticity [Ec]       = 5000 √fckN/mm2(Cl. 6.2.3.1, IS 456:2000)

= 25000 N/mm2   for M25

Poisson’s Ratio [U]   	= 0.2

Unit Weight   		= 25 KN/m3

Characteristic Strength [ƒck]  = 25 N/mm2 for M25

The structural design strength is derived from the characteristic strength multiplied by a coefficient 0.67 and divided by the material partial safety factor. The partial factor for concrete in flexure and axial load is 1.5.

### Reinforcement Steel

Characteristic strength of high yield steel is taken as 500 MPa for main longitudinal rebar and 500 Mpa for shear rebar and material partial safety factor is to be 1.15.

Modulus of Elasticity [Es]  	= 2x105 N/mm2

Poisson’s Ratio [U]   	= 0.3

## Section Properties

•Preliminary Size of Members

The preliminary sizes of Beam, Column, and Slab were chosen based on experience.

| Beam | : 355 mm x 355 mm M25 |
| --- | --- |
| Column | : 355 mm x 355 mm M25 |
| Floor Slab Thickness | : 125 mm M25 |
| Staircase Slab Thickness | : 125 mm M25 |

During the analysis, beam and column are modeled as frame elements whereas slabs are modeled as area element.

## Loadings

The following considerations are made during the loading on the structural model:

- The loads distributed over the area are imposed on the area element and the loads distributed over the length are imposed on the frame elements whenever possible.

- Where such loading is not possible, equivalent conversion to different loading distribution is carried to load the model near the real case as far as possible.

- For lateral load, necessary calculations are performed to comply with the requirements of NBC 105: 2025.

### Load Cases

The following load cases are used for the loading during analysis.

Table 1: Load Cases

| Load Name | Load Type | Description | Unit | Remarks |
| --- | --- | --- | --- | --- |
| DL | Dead | Self-wt of the structure | kN/m2 |  |
| WL | Dead | Wall Load | kN/m | on beam |
| FFL | Dead | Floor Finish Load | kN/m2 | on slab |
| PWL | Dead | Partition Wall Load | kN/m2 | on slab |
| LL | Live | Imposed Load | kN/m2 | on slab |
| Stair Live | Live | Imposed Load | kN/m2 | on Staircase |
| Stair steps | Dead | Staircase steps load | kN/m2 | on Staircase |
| Water tank | Live | Imposed Load | kN/m2 | on terrace/roof |
| EQX | Earthquake | As per NBC 105 |  | X+0.05Y |
| EQY | Earthquake | As per NBC 105 |  | Y+0.05X |

### Load Combination

The following load combinations are specified as per NBC 105:2025:

Static Load Combination:

1.2DL +1.5 LL

Seismic Load Combination:

DL +LL ± EQx / EQy

0.9 DL ± EQx / EQy

The following load combinations are used during analysis

#### Table 2: Load Combination

| S.N | Name | Type |
| --- | --- | --- |
| 1 | 1.2DL + 1.5LL | 1.2DL + 1.5LL |
| 2 | DL + YLL + EQx | DL + YLL + EQx |
| 3 | DL + YLL - EQx | DL + YLL - EQx |
| 4 | DL + YLL+ EQy | DL + YLL+ EQy |
| 5 | DL + YLL-EQy | DL + YLL-EQy |
| 6 | 0.9DL + EQx | 0.9DL + EQx |
| 7 | 0.9DL - EQx | 0.9DL - EQx |
| 8 | 0.9DL + EQy | 0.9DL + EQy |
| 9 | 0.9DL - EQy | 0.9DL - EQy |

# MODELLING OF THE STRUCTURE

The structure is modelled in ETABS version 22, different data for modelling are shown in the following sections.

## Three Dimensional views of Model

3D View

## Joint ID

## Section Property

### Beam and Columns

Elevation View A

Elevation View B

Elevation View C

Elevation View D

Elevation View E

### Slab

## Application of loads

The loads on the building are based on Indian codes of Practices. The unit weight of different structural and non-structural elements are derived from IS 875 Part 1 and presented in Table 3. The load calculations are based on actual measured drawings. The self-weight of beams, columns and slabs are calculated by the program. Similarly the imposed loads are applied on the slab as area load in kN/m2 and values of imposed loads are tabulated in Table 5 below.

- The weight of infill walls are calculated and applied on beams as line weight in kN/m.

- Partition wall load are assigned as uniformly distributed area load in slab as area load in kN/m2.

- Floor finishing load are assigned as area load in slab.

- Single type of Live load is assigned in each panel of slab.

- A frame load is applied as parapet loading on the exterior frame of the roof level.

- The roof is assumed accessible and loaded with roof live load as per Indian Standard, IS 875 -1987(part2) but this load is not considered during seismic load.

### Dead Loads

#### Unit Weight (Dead Load)

Dead loads for analysis are calculated as per Indian Standard, IS 875 -1987 (part-I). Unit weights of different material used are given below.

Table 3: Unit Weight of Materials

| S.N | Type | Value |
| --- | --- | --- |
| 1 | Reinforced Concrete | 25 kN/m3 |
| 2 | Brick Masonry | 19.2 kN/m3 |
| 3 | Screed | 22.0 kN/m3 |
| 4 | Marble | 26.7 kN/m3 |
| 5 | Plaster | 20.4 kN/m3 |
| 7 | Steel Rebar | 78.6 KN/m3 |

#### Dead Load Calculation

- Unit Weights of materials

Brick masonry			             19.2kN/m3

Screed			            22kN/m3

Marble				26.7kN/m3

Reinforced Concrete				25kN/m3

Cement plaster					20.4kN/m3

- Heights of Beams, Walls & Parapet Walls

Depth of Beam 			           0.355m

Height of Building					3.2 at GF & FF and 2.896m remaining

Height of  Parapet Wall 				1.000m Wall thickness 0.115m

- Dead Loads of Walls

- Dead load of 230 mm thick wall (without opening) ht. upto beam 12.56kN/m for GF & FF and 11.22KN/m for SF and above.

Dead load of 230 mm thick wall (without opening) ht. upto slab 13.57kN/m for GF & FF and 12.23KN/m for SF and above.

Dead load of 230 mm thick wall (with opening) ht. upto beam 9.42kN/m for GF & FF and 8.41KN/m for SF and above.

Dead load of 230 mm thick wall (with opening) ht. upto slab 10.18kN/m for GF & FF and 9.17KN/m for SF and above.

- Dead load of 115 mm thick wall (without opening) ht. upto beam 6.28kN/m for GF & FF and 5.61KN/m for SF and above.

Dead load of 115 mm thick wall (without opening) ht. upto slab 6.79kN/m for GF & FF and 6.11KN/m for SF and above.

Dead load of 115 mm thick wall (with opening) ht. upto beam 4.71kN/m for GF & FF and 4.21KN/m for SF and above.

Dead load of 115 mm thick wall (with opening) ht. upto slab 5.09kN/m for GF & FF and 4.58KN/m for SF and above.

- Dead load of  parapet wall (115 mm thick) 2.02kN/m

- Floor Loads

Thickness of slab		0.125m

Thickness of tile with plaster		0.025m

Thickness of Mosaic		0.020m

Thickness of Marble		0.020m

Thickness of Screed		0.040m

Thickness of Cement Plaster		0.0125m

Dead load of structural slab		3.125kN/m2

Dead load of Tile		0.510kN/m2

Dead load of Mosaic		0.462kN/m2

Dead load of Marble		0.534kN/m2

Dead load of screed		0.840kN/m2

Dead load of Cement Plaster		0.255kN/m2

Total dead load of Floor Finishes (Tile)		1.350kN/m2

Total dead load of Floor Finishes (Mosaic)		1.302kN/m2

Total dead load of Floor Finishes (Marble)		1.374kN/m2

Total dead load of Floor Finishes (Cement Punning)		1.095kN/m2

Dead load of light partition walls					      1.91kN/m2

Wall load

### Live Load

The magnitude of live load depends upon the type of occupancy of the building. These are to be chosen from code IS875:1987(part II) for various occupancies. The live load distribution varies with time. Hence each member is designed for worst combination of dead load and live loads. Live loads for residential building are given below.

Table 5: Live Load for Residential Building

| S.N | Area type | Load | Unit |
| --- | --- | --- | --- |
| 1 | Rooms | 2.0 | kN/m2 |
| 2 | Terrace (Accessible) | 1.5 | kN/m2 |
| 3 | Terrace (Inaccessible) | 0.75 | kN/m2 |
| 4 | Balcony, Staircase and Passage | 3.0 | kN/m2 |
| 5 | Toilet | 2.0 | kN/m2 |

Live Load

### Floor Finish

FF Load

## Seismic Loading

The seismic load is applied to the building with user defined coefficient in ETABS 2022 calculated as per NBC 105-2025. This load case is assumed static linear and all the necessary data and calculation are given as per the following conditions.

To determine the seismic load, it is considered that the site lies in the seismic zone Gorkha according to NBC 105:2025. The soil type is considered as soft with 5% damping to determine average response acceleration. The building is analyzed as moment resisting frame with consideration of infill wall. Therefore the fundamental time period T1 .

Detail Calculation is presented in Table 7 below.

The seismic weight is determined based on the following mass source. [Table

6, Cl.6.0.0, NBC 105: 2025]

### Table 6: Mass Source for Seismic Load

| S.N | Load Type | Scale Factor |
| --- | --- | --- |
| 1 | Dead Load | 1 |
| 2 | Live Load | 0.3 |
|  |  | 0.6 for storage purpose |

# ANALYSIS OUTPUT

The analysis results are discussed in this chapter. Seismic coefficient method is used. The major discussion are focused on the eccentricity, story shear, inter story drift, maximum displacement and base shear along two orthogonal directions. The column and beam size and reinforcement are then designed for maximum forces.

## Seismic Coefficient

The fundamental time period of the building as per NBC 105:2025

| FUNDAMENTAL TIME PERIOD AS PER NBC 105: 2025 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NBC 105: 2025 CL: 5.1.2 (Page 28) |  |  |  |  |  |  |  |  |  |  |  |  |
| kt | = | 0.075 |  | moment resisting concrete frame |  |  |  |  |  |  |  |  |
| H | = | 15.0876 |  | m |  |  |  |  |  |  |  |  |
| T1 | = | 0.5742 |  | seconds |  |  |  |  |  |  |  |  |
| The approximate fundamental time period as per |  |  |  |  |  |  |  |  |  |  |  |  |
| NBC 105: 2025 CL: 5.1.3 (Page 29) |  |  |  |  |  |  |  |  |  |  |  |  |
| T1 | = | 0.7177 |  | seconds |  |  |  |  |  |  |  |  |
| Tr |  | 0.52 |  |  |  |  |  |  |  |  |  |  |
| Ta | = | 0.5 |  |  |  |  |  |  | K | = | 1.108845 |  |
| BASE SHEAR COEFFICIENT AS PER NBC 105: 2025 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Z |  | = | 0.35 | Kathmandu |  |  |  |  | Table 4-5 |  |  |
|  |  |  | I |  | = | 1 | Residential |  |  |  |  | Table 4-6 |  |  |
|  |  |  | Ch(T) |  | = | 2.25 |  |  |  |  |  | Clause: 4.1.2 |  |  |
|  |  |  | T |  | = | 0.7177 | Sec |  |  |  |  | (Ta<T<Tc) |  |  |
|  |  |  | Soil Type |  | = | D |  |  |  |  |  | Table 4-4 |  |  |
|  |  |  | a |  | = | 2.25 | check table 4.1 |  |  |  |  | Table 4-1 |  |  |
|  |  |  | Ta |  | = | 0.5 |  |  |  |  |  | Table 4-1 |  |  |
|  |  |  | Tc |  | = | 2 |  |  |  |  |  | Table 4-1 |  |  |
|  |  |  | K |  | = | 0.8 |  |  |  |  |  | Table 4-1 |  |  |
| For Ultimate Limit State |  |  | C(T) = Ch(T) Z I |  | = | 0.7875 |  |  |  |  |  | Clause: 4.1.1 |  |  |
| For Serviceability Limit State |  |  | Cs(T) = 0.2 C(T) |  | = | 0.1575 |  |  |  |  |  | Clause: 4.2 |  |  |
| Horizontal Base Shear Coefficient for Ultimate Limit State (Clause: 6.1.1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Rm |  | = | 4 |  |  |  |  |  |  |  |  |
|  |  |  | Wu |  | = | 1.5 |  |  |  |  |  |  |  |  |
|  |  |  | Cd(T1) |  | = | 0.1313 |  |  |  |  |  |  |  |  |
| Horizontal Base Shear Coefficient for Serviceability Limit State (Clause: 6.1.2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Ws |  | = | 1.25 |  |  |  |  |  |  |  |  |
|  |  |  | Cd(T1) |  | = | 0.1260 |  |  |  |  |  |  |  |  |

## Acceptability Criteria:

### Inter-story drift:

Inter storey drift (Load Case EQx)

Inter storey drift (Load Case EQy)

As per cl. no. 5.5.3 of NBC 105:2025, The ratio of the inter-story deflection to the corresponding story height shall not exceed:

0.025 at ultimate limit state

0.006 at serviceability limit state

### Storey Displacement:

Storey Displacement: EQx

Story displacement: EQy

## Modal time period and mass participation

NBC 105:1994 clause 11.2 states that a sufficient number of modes shall be considered to ensure that at least 90 percent of the mass is participating in the direction under consideration. Analysis was carried out for first 10 modes so that the mass participation satisfies this criterion in both orthogonal directions. Table shows time period and mass participation ratio for all modes.

| TABLE: Modal Participating Mass Ratios |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case | Mode | Period | UX | UY | UZ | SumUX | SumUY | SumUZ | RX | RY | RZ |  | SumRX |  | SumRY |  | SumRZ |  |
|  |  | sec |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Modal | 1 | 1.131 | 0.6939 | 0.0002 | 0 | 0.6939 | 0.0002 | 0 | 1.741E-05 | 0.179 | 0.1385 |  | 1.741E-05 |  | 0.179 |  | 0.1385 |  |
| Modal | 2 | 1.095 | 0.0048 | 0.8278 | 0 | 0.6987 | 0.828 | 0 | 0.1677 | 0.0003 | 0.0152 |  | 0.1677 |  | 0.1793 |  | 0.1537 |  |
| Modal | 3 | 1.01 | 0.1451 | 0.019 | 0 | 0.8438 | 0.8469 | 0 | 0.0043 | 0.0059 | 0.6872 |  | 0.172 |  | 0.1852 |  | 0.8409 |  |
| Modal | 4 | 0.344 | 0.0793 | 4.289E-05 | 0 | 0.9231 | 0.847 | 0 | 0.0002 | 0.5475 | 0.017 |  | 0.1722 |  | 0.7327 |  | 0.8579 |  |
| Modal | 5 | 0.34 | 0.0001 | 0.0894 | 0 | 0.9232 | 0.9364 | 0 | 0.6505 | 0.0005 | 0.0009 |  | 0.8227 |  | 0.7332 |  | 0.8589 |  |
| Modal | 6 | 0.31 | 0.0136 | 0.0011 | 0 | 0.9368 | 0.9375 | 0 | 0.0084 | 0.0992 | 0.0764 |  | 0.8311 |  | 0.8324 |  | 0.9353 |  |
| Modal | 7 | 0.194 | 0.0011 | 0.0389 | 0 | 0.938 | 0.9764 | 0 | 0.0805 | 0.0023 | 0.0012 |  | 0.9116 |  | 0.8346 |  | 0.9364 |  |
| Modal | 8 | 0.191 | 0.041 | 0.0011 | 0 | 0.9789 | 0.9775 | 0 | 0.0022 | 0.0829 | 0.0012 |  | 0.9137 |  | 0.9175 |  | 0.9376 |  |
| Modal | 9 | 0.183 | 9E-07 | 0.0018 | 0 | 0.9789 | 0.9793 | 0 | 0.0038 | 0 | 0.0406 |  | 0.9175 |  | 0.9175 |  | 0.9782 |  |
| Modal | 10 | 0.139 | 0.0094 | 0.0033 | 0 | 0.9883 | 0.9826 | 0 | 0.0137 | 0.0371 | 0.006 |  | 0.9312 |  | 0.9546 |  | 0.9841 |  |
| Modal | 11 | 0.136 | 0.0039 | 0.014 | 0 | 0.9922 | 0.9966 | 0 | 0.0586 | 0.0158 | 0.0001 |  | 0.9898 |  | 0.9704 |  | 0.9842 |  |
| Modal | 12 | 0.128 | 0.005 | 0.0006 | 0 | 0.9972 | 0.9973 | 0 | 0.0028 | 0.0216 | 0.0125 |  | 0.9926 |  | 0.992 |  | 0.9967 |  |

| TABLE: Modal Load Participation Ratios |  |  |  |  |
| --- | --- | --- | --- | --- |
| Case | ItemType | Item | Static | Dynamic |
|  |  |  | % | % |
| Modal | Acceleration | UX | 100 | 99.72 |
| Modal | Acceleration | UY | 100 | 99.73 |
| Modal | Acceleration | UZ | 0 | 0 |

# ANALYSIS OF THE STRUCTURE

## Axial Force Diagram

## Shear Force Diagram

## Bending Moment Diagram

## Reinforcement from Etabs

### Columns

Column Along Grid A-A

Column Along Grid B-B

Column Along Grid C-C

Column Along Grid D-D

Column Along Grid E-E

### Beams

Beam (First Floor)

Beam (Second Floor)

Beam (Third Floor)

Beam (Fourth Floor)

Beam (Roof Floor)

# DESIGN OF STRUCTURAL ELEMENTS

## Design of columns

ETABS Concrete Frame Design

IS 456:2000 + IS 13920:2016  Column Section Design   (Summary)

Column Element Details

| Level | Element | Unique Name | Section ID | Combo ID | Station Loc | Length (mm) | LLRF | Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Story2 | C17 | 145 | C 14"*14" | DL +YLL-EQxULS | 0 | 3200.4 | 1 | Ductile Frame |

Section Properties

| b (mm) | h (mm) | dc (mm) | Cover (Torsion) (mm) |
| --- | --- | --- | --- |
| 355.6 | 355.6 | 78 | 50 |

Material Properties

| Ec (MPa) | fck (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 25000 | 25 | 1 | 500 | 500 |

Design Code Parameters

| ɣC | ɣS |
| --- | --- |
| 1.5 | 1.15 |

Axial Force and Biaxial Moment Design For Pu , Mu2 , Mu3

| Design Pu kN | Design Mu2 kN-m | Design Mu3 kN-m | Minimum M2 kN-m | Minimum M3 kN-m | Rebar Area mm² | Rebar % % |
| --- | --- | --- | --- | --- | --- | --- |
| 517.1834 | 10.3437 | -153.132 | 10.3437 | 10.3437 | 3378 | 2.67 |

Axial Force and Biaxial Moment Factors

|  | K Factor Unitless | Length mm | Initial Moment kN-m | Additional Moment kN-m | Minimum Moment kN-m |
| --- | --- | --- | --- | --- | --- |
| Major Bend(M3) | 2.866043 | 2844.8 | -42.9574 | 45.7386 | 10.3437 |
| Minor Bend(M2) | 0.921148 | 2844.8 | 1.4251 | 0 | 10.3437 |

Axial Compression Check for Seismic Design per IS 13920:2016 Section 7.1

| Limit Compression Factor Unitless | Limit Compression Strength kN | Compression D/C Ratio Unitless | Status |
| --- | --- | --- | --- |
| 0.4 | 1264.5136 | 0.408998 | OK |

Shear Design for Vu2 , Vu3

|  | Shear Vu kN | Shear Vc kN | Shear Vs kN | Shear Vp kN | Rebar Asv /s mm²/m |
| --- | --- | --- | --- | --- | --- |
| Major, Vu2 | 65.1121 | 113.8675 | 39.4853 | 50.2787 | 394.16 |
| Minor, Vu3 | 44.9928 | 113.0202 | 39.4853 | 44.9928 | 394.16 |

Joint Shear Check/Design

|  | Joint Shear Force kN | Shear VTop kN | Shear Vu,Tot kN | Shear Vc kN | Joint Area cm² | Shear Ratio Unitless |
| --- | --- | --- | --- | --- | --- | --- |
| Major Shear, Vu2 | N/N | N/N | N/N | N/N | N/N | N/N |
| Minor Shear, Vu3 | N/N | N/N | N/N | N/N | N/N | N/N |

(1.4) Beam/Column Capacity Ratio

| Major Ratio | Minor Ratio |
| --- | --- |
| N/N | N/N |

Additional Moment Reduction Factor k (IS 39.7.1.1)

| Ag cm² | Asc cm² | Puz kN | Pb kN | Pu kN | k Unitless |
| --- | --- | --- | --- | --- | --- |
| 1264.5 | 33.8 | 2689.3087 | 393.5624 | 517.1834 | 0.946152 |

Additional Moment (IS 39.7.1)

|  | Consider Ma | Length Factor | Section Depth (mm) | KL/Depth Ratio | KL/Depth Limit | KL/Depth Exceeded | Ma Moment (kN-m) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Major Bending (M3 ) | Yes | 0.889 | 355.6 | 22.928 | 12 | Yes | 48.3417 |
| Minor Bending (M2 ) | Yes | 0.889 | 355.6 | 7.369 | 12 | No | 0 |

Notes:

N/A: Not Applicable

N/C: Not Calculated

N/N: Not Needed

Table 1:  Summary of Design of Column

| S.N. | Grid Location | Column Size | Ground Floor | First Floor | Second Floor | Third Floor | Top Floor | Stirrups |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | A6 | 350 x 350 | 8-20Ø | 8-20Ø | 4-20Ø 4-16Ø | 4-20Ø 4-16Ø |  |  |
| 2 | B5 | 350 x 350 | 4-25Ø 4-20Ø | 4-25Ø 4-20Ø | 8-20Ø | 4-20Ø 4-16Ø | 8-16Ø | mm C/C (at top and bottom) & @ 150 mm C/C (at mid height) in all columns |
| 3 | C3 | 350 x 350 | 8-25Ø | 8-25Ø | 4-25Ø 4-20Ø | 8-20Ø | 4-20Ø 4-16Ø |  |
| 4 | C4 | 350 x 350 | 4-25Ø 4-20Ø | 4-25Ø 4-20Ø | 8-20Ø | 4-20Ø 4-16Ø | 4-20Ø 4-16Ø |  |
| 5 | D6 | 350 x 350 | 8-25Ø | 8-25Ø | 4-25Ø 4-20Ø | 8-20Ø |  |  |
| 6 | D5 | 350 x 350 | 4-25Ø 4-20Ø | 4-25Ø 4-20Ø | 8-20Ø | 4-20Ø 4-16Ø | 4-20Ø 4-16Ø |  |
| 7 | D4 | 350 x 350 | 4-25Ø 4-20Ø | 4-25Ø 4-20Ø | 8-20Ø | 4-20Ø 4-16Ø | 4-20Ø 4-16Ø |  |
| 8 | D3 | 350 x 350 | 4-25Ø 4-20Ø | 4-25Ø 4-20Ø | 8-20Ø | 4-20Ø 4-16Ø | 4-20Ø 4-16Ø |  |
| 9 | D2 | 350 x 350 | 4-25Ø 4-20Ø | 4-25Ø 4-20Ø | 8-20Ø | 4-20Ø 4-16Ø | 4-20Ø 4-16Ø |  |
| 10 | E6 | 350 x 350 | 4-25Ø 4-20Ø | 8-20Ø | 4-20Ø 4-16Ø | 4-20Ø 4-16Ø |  |  |
| 11 | E5 | 350 x 350 | 4-25Ø 4-20Ø | 4-25Ø 4-20Ø | 8-20Ø | 4-20Ø 4-16Ø | 8-16Ø |  |
| 12 | E4 | 350 x 350 | 8-20Ø | 8-20Ø | 4-20Ø 4-16Ø | 4-20Ø 4-16Ø | 4-20Ø 4-16Ø |  |
| 13 | E3 | 350 x 350 | 8-20Ø | 8-20Ø | 4-20Ø 4-16Ø | 4-20Ø 4-16Ø | 4-20Ø 4-16Ø |  |
| 14 | E2 | 350 x 350 | 4-25Ø 4-20Ø | 8-20Ø | 4-20Ø 4-16Ø | 8-16Ø | 8-16Ø |  |

## Design of beams

ETABS Concrete Frame Design

IS 456:2000 + IS 13920:2016   Beam Section Design   (Summary)

Beam Element Details

| Level | Element | Unique Name | Section ID | Combo ID | Station Loc | Length (mm) | LLRF | Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Story2 | B37 | 165 | B 9" *14" | DL +YLL-EQxULS | 177.8 | 3302 | 1 | Ductile Frame |

Section Properties

| b (mm) | h (mm) | bf (mm) | ds (mm) | dct (mm) | dcb (mm) |
| --- | --- | --- | --- | --- | --- |
| 228.6 | 355.6 | 228.6 | 0 | 25 | 25 |

Material Properties

| Ec (MPa) | fck (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27386.13 | 30 | 1 | 500 | 500 |

Design Code Parameters

| ɣC | ɣS |
| --- | --- |
| 1.5 | 1.15 |

Factored Forces and Moments

| Factored Mu3 kN-m | Factored Tu kN-m | Factored Vu2 kN | Factored Pu kN |
| --- | --- | --- | --- |
| -89.605 | 8.3286 | 105.8438 | 0 |

Design Moments, Mu3 & Mt

| Factored Moment kN-m | Factored Mt kN-m | Positive Moment kN-m | Negative Moment kN-m |
| --- | --- | --- | --- |
| -89.605 | 12.5201 | 0 | -102.1252 |

Design Moment and Flexural Reinforcement for Moment, Mu3 & Tu

|  | Design -Moment kN-m | Design +Moment kN-m | -Moment Rebar mm² | +Moment Rebar mm² | Minimum Rebar mm² | Required Rebar mm² |
| --- | --- | --- | --- | --- | --- | --- |
| Top (+2 Axis) | -102.1252 |  | 876 | 0 | 876 | 219 |
| Bottom (-2 Axis) |  | 0 | 438 | 0 | 20 | 438 |

Shear Force and Reinforcement for Shear, Vu2 & Tu

| Shear Ve kN | Shear Vc kN | Shear Vs kN | Shear Vp kN | Rebar Asv /s mm²/m |
| --- | --- | --- | --- | --- |
| 109.78 | 0 | 168.073 | 83.7902 | 1408.79 |

Torsion Force and Torsion Reinforcement for Torsion, Tu & VU2

| Tu kN-m | Vu kN | Core b1 mm | Core d1 mm | Rebar Asvt /s mm²/m |
| --- | --- | --- | --- | --- |
| 8.3286 | 105.8438 | 198.6 | 325.6 | 717.23 |

Table 2: Beam reinforcement details

| Beam Reinforcement Detail |  |  |  |  |
| --- | --- | --- | --- | --- |
| S.N. | Type of beam | Size of beam | Direction | Reinforcement |
|  |  |  |  | At support |  |  | At center |  |
|  |  |  |  | Top |  | Bottom | Top | Bottom |
| 1 | Tie beam | 230*350 | X-dir | 3-16ɸ Th |  | 3-16ɸTh | 2-16ɸ Th | 3-16ɸTh |
|  |  |  | Y-dir | 3-16ɸ Th |  | 3-16ɸTh | 2-16ɸ Th | 3-16ɸTh |
| 2 | Ground Floor | 230*350 | X-dir | 2-16ɸTh+1-12ɸEx) +2-16ɸ (Ex) |  | 3-16ɸ Th | 2-16ɸ Th | 3-16ɸTh |
|  |  |  | Y-dir | 2-16ɸTh+1-12ɸEx) +2-16ɸ (Ex) |  | 3-16ɸ Th | 2-16ɸ Th | 3-16ɸTh |
| 3 | Main beam First Floor, Second, Floor, Third Floor | 230*350 | X-dir | 2-16ɸTh+1-12ɸEx) +2-16ɸ (Ex) |  | 3-16ɸ Th | 2-16ɸ Th | 3-16ɸTh |
|  |  |  | Y-dir | 2-16ɸTh+1-12ɸEx) +2-16ɸ (Ex) |  | 3-16ɸ Th | 2-16ɸ Th | 3-16ɸTh |
| 4 | Main beam Top Floor | 230*350 | X-dir | 2-12ɸTh+1-16ɸ(Ex) |  | 3-12ɸ Th | 2-12ɸ Th | 3-12ɸ Th |
|  |  |  | Y-dir | 2-12ɸTh+1-16ɸ(Ex) |  | 3-12ɸ Th | 2-12ɸ Th | 3-12ɸ Th |

## Design of Slab

| Design of Two way Slab by Limit State Method |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| (As per IS 456:2000) |  |  |  |  |  |
| 1 | Data |  |  |  |  |
|  | Longer span of slab, ly | 4.521 | m |  |  |
|  | Shorter span of slab, lx | 3.454 | m |  |  |
|  | Materials used |  |  |  |  |
|  | Concrete | M20 | 20 | N/mm2 |  |
|  | Steel | Fe500 | 500 | N/mm2 |  |
| 2 | Effective span |  |  |  |  |
|  | Longer span of slab, ly | 4.344 | m |  |  |
|  | Shorter span of slab, lx | 3.277 | m |  |  |
|  | ly/lx | 1.326 |  |  |  |
|  | type of slab | <2 two way slab |  |  |  |
|  | Support Condition : |  |  |  |  |
|  | One Short Edge Discontinuous |  |  |  |  |
| 3 | Calculation of loads |  |  |  |  |
|  | i) Dead load |  |  |  |  |
| i | Slab thickness | 125 | mm |  |  |
|  | Self weight of concrete | 25 | KN/m3 |  |  |
|  | Dead load of slab = 25 * 0.125 | 3.125 | KN/m2 |  |  |
| ii | Screeding thickness | 30 | mm |  |  |
|  | Self weight of screeding | 20.4 | KN/m3 |  |  |
|  | Dead load of screeding = 20.4 * 0.03 | 0.612 | KN/m2 |  |  |
| iii | marble flooring thickness | 19 | mm |  |  |
|  | Self weight of marble | 27 | KN/m3 |  |  |
|  | Dead load of marble = 27 * 0.019 | 0.513 | KN/m2 |  |  |
| iv | Cement plaster thickness | 12.5 | mm |  |  |
|  | Self weight of plaster | 20.4 | KN/m3 |  |  |
|  | Dead load of plaster = 20.4 * 0.0125 | 0.255 | KN/m2 |  |  |
|  | Total dead load on slab | 4.505 | KN/m2 |  |  |
|  | ii) Live load |  |  |  |  |
|  | For Residential Building | 2 | KN/m2 |  |  |
|  | Total design load (DL+LL) | 6.505 | KN/m2 |  |  |
|  | Factored load = 1.5 * design load | 9.7575 | KN/m2 |  |  |
| 4 | Calculation of moments |  |  |  |  |
|  | As per code, | Shorter Direction |  | Longer Direction |  |
|  | -ve moment at support | Support, αx1 | 0.052 | Support, αy1 | 0.037 |
|  | +ve moment at mid | mid, αx2 | 0.040 | mid, αy2 | 0.028 |
|  | Along shorter span, |  |  |  |  |
|  | Mx = αx1*wu*lx | 6.599 | KNm |  |  |
|  | My = αy1*wu*lx | 4.693 | KNm |  |  |
|  | Along longer span, |  |  |  |  |
|  | Mx = αx2*wu*lx | 5.012 | KNm |  |  |
|  | My = αy2*wu*lx | 3.552 | KNm |  |  |
| 5 | Depth Calculation |  |  |  |  |
|  | Minimum depth of slab is calculated considering maximum bending moment |  |  |  |  |
|  | Maxm bending moment | 6.599 |  |  |  |
|  | Depth required, |  |  |  |  |
|  | d=sqrt(Mu/(0.36*xumax/d)*(1-0.42*xumax/d)*b*fck) |  |  |  |  |
|  | overall depth, D=d+φ/2+clear cover) |  | mm |  |  |
|  | For safe design, adopting depth of slab | 125 | mm |  |  |
|  | Effective depth | 96 | mm |  |  |
| 6 | Reinforcement Calculation | Main Bar | Dist. Bar |  |  |
| provide the reinforcement of dia |  | 10 | 8 | mm |  |
| S. No. | Description | Shorter Direction |  | Longer Direction |  |
|  |  | Support | mid | Support | mid |
| 1 | αx & αy | 0.052 | 0.040 | 0.037 | 0.028 |
| 2 | BM | 6.60 | 5.01 | 4.69 | 3.55 |
| 3 | Ast required | 149.81 | 112.75 | 105.40 | 79.25 |
| 4 | Ast min | 150.00 | 150.00 | 150.00 | 150.00 |
| 5 | Spacing reqd | 523.81 | 523.81 | 335.24 | 335.24 |
| 6 | Spacing prov. | 150.00 | 150.00 | 150.00 | 150.00 |
| 7 | Ast provided | 523.81 | 523.81 | 335.24 | 335.24 |
| 8 | % of steel provided | 0.42 | 0.42 | 0.27 | 0.27 |
| 7 | Check |  |  |  |  |
| 7.1 | Minimum reinforcement |  |  |  |  |
|  | Min Ast = 0.12% bD | 150 | mm2 |
| 7.2 | Check for deflection |  |  |  |  |
|  | From deflection control criteria |  |  |  |  |
|  | lx/d < (l/d)basic*αβγ | where |  |  |  |
|  | lx/d | 31.2085714 |  |  |  |
|  | α = modification factor for tension reinforcement |  |  |  | fig 4 |
|  | β = modification factor for compression reinf. |  | 1 |  | fig 5 |
|  | γ = reduction factor for ratios of span to eff. depth in flanged beam |  | 1 |  | fig 6 |
|  | (l/d)basic = | 23 |  |  | of IS 456 |
|  | For α, |  |  |  |  |
|  | fs = 0.58*fy*(Ast required/Ast provided) | 83.045 | N/mm2 |  |  |
|  | % of tensional reinforcement | 0.419 |  |  |  |
|  | α | 2 |  |  |  |
|  | now, (l/d)basic*αβγ | 46 |  | OK Safe |  |
|  | Since, lx/d < (l/d)basic*αβγ | SAFE |  |  |  |
| 7.3 | Check for Shear |  |  |  |  |
|  | Shear is checked along short span. |  |  |  |  |
|  | Maximum shear force, Vumax = w*lx/2 | 19.354 | KN |  |  |
|  | Normal shear stress, τv = V/bd | 0.184 | N/mm2 |  |  |
|  | % of steel provided = 100*Ast provided/bD | 0.249 |  |  |  |
|  | Shear capacity of section without reinforcement |  |  |  |  |
|  | From table 19, |  |  |  |  |
|  | τc | 0.360 | N/mm2 |  |  |
|  | Shear strength, τ'c = k*τc |  |  |  |  |
|  | For slab thickness 125mm, k | 1.3 |  |  |  |
|  | Shear strength, τ'c | 0.467 | N/mm2 |  |  |
|  | Since, τ'c > τv | OK Safe |  |  |  |
| 8 | Results |  |  |  |  |
|  | Overall depth of slab = | 125 | mm |  |  |
|  | Main bar along short span at support | 10mm ø @ 150mm c/c |  |  |  |
|  | Main bar along short span at mid | 10mm ø @ 150mm c/c |  |  |  |
|  | Distribution Bar along long span at support | 8mm ø @ 150mm c/c |  |  |  |
|  | Distribution bar along long span at mid | 8mm ø @ 150mm c/c |  |  |  |

## Design of Staircase

  - Design of Footing

- Isolated, Strap and Combined Footing  is designed

- Soil Subgrade Modulus

Factor of saftely	:	3Soil g Capacity	:	120 KN/m2

Allowable Settlement	:	25 mm

Soil Spring Modulus		=	3 x 150 x (1000/20)

=	225000 KN/m3

Preliminary slab thickness 	= 	457.2 mm (18”)

Design of Strap Beam (A3-E3)

| Joint level | Unfactored load | Footing size(ft) | Depth of footing pad | Reinforecement bar |
| --- | --- | --- | --- | --- |
| A2 | 445.98 | 7X7 | 450mm | 12 mm 5" c/c Both ways |
| A3 | 348.72 | 6X6 |  |  |
| A4 | 294.25 | 6X6 |  |  |
| A5 | 193.88 | 5.5X5.5 |  |  |
| B1 | 435.23 | 7X7 |  |  |
| B2 | 9.98 | 6X6 |  |  |
| B3 | 569.28 | 8X8 |  |  |
| B4 | 486.48 | 8X8 |  |  |
| B5 | 343.82 | 6X6 |  |  |
| C1 | 229.40 | 6X6 |  |  |
| C2 | 362.78 | 7X7 |  |  |
| C3 | 247.81 | 6X6 |  |  |
| C4 | 304.65 | 6X6 |  |  |
| C5 | 233.59 | 6X6 |  |  |

# CONCLUSIONS AND RECOMMENDATION

Residential building with simple configuration has been fully designed using a computer program, ETABSv18. All the required design details of slab, beam, column and footing and staircase have been presented. Possible best-approaches have been adopted for most-economical design yet fulfilling all the requirements for important building. Due considerations have been given to the probable increase in load and strength as a partial factor of safety. Also, double tie-beams, on regular tie-beam at plinth level and one at top level of footing-pad is specified as the building is being constructed in high seismicity area. During construction, it is mandatory to followIS13920, regarding provision of stirrups and ties, development lengths, splices and other detailing. Band at sill and lintel are provided to increase lateral strength of wall and to improve opening weakness on wall. Strict Control over quality of materials and workmanship is required for expected performance of building in future. Hence, following shall be considered during construction works to obtain expected results.

A. General:

a. All works to be carried out in accordance with current best practice, Building Regulations, the project specification and relevant Nepal Building Code (NBC), Indian Standards and Codes of Practice. Materials and components to be appropriate for their intended use.

b. The construction-works shall only be carried-out by trained mason with supervision of Engineer.

c. During construction, the contractor shall be responsible for maintaining the structure in a stable condition and ensuring no part shall be damaged under construction activities.

d. Workmanship and materials are to be in accordance with the relevant current Standards including all amendments and the local statutory authorities, except where varied by the contract document.

e. All coarse aggregate used shall be crushed stone aggregate. The nominal size of coarse-aggregate for RC bands and splints shall not exceed 12.5mm.

f. Clean sand, with minimum silt and free from clay and organic materials shall be used.

g. Ordinary Portland cement conforming to IS 269:1976 shall be used for all cement works.

h. At least 48 hours’ notice shall be provided for all engineering inspections.

B. Structural:

1. Cast-In-Situ concrete/micro-concrete shall have minimum 28 days’ compressive cube strength of 20N/mm² for all structural members unless otherwise stated.

2. The concrete compressive strength shall be measured on 150*150*150mm cube at 28 days, for various structural elements.

3. Reinforcing steel shall be, TMT rebar with ultimate strain not less than 14.5% can also be used.

4. Cover to main reinforcing steel be in accordance with IS 456:1978 & as specified in the structural drawings.

5. Clear Cover of Concrete shall not be less than that given below:

a. Concrete surface at soil = 50mm

b. Concrete on PCC, Bricks, STONE, etc. = 25mm

6. Unless otherwise specified, all horizontal & vertical construction joints shall be roughened.

7. A minimum of 48 hours’ notice shall be given to the Engineer before applying plaster, concrete/microconcrete is poured, in order that the formwork and/or reinforcement may be inspected.

8. All R.C.C work shall be continuously cured for 14-days.

9. All cement plaster works shall be continuously cured for 7 days.

10. Any damage to surface during erection/construction is to be made good.

# REFERENCES

a.	IS code 456-2000: 	Plain and Reinforced Concrete code of Practice

b.	IS code 1893 (part I)-2002: 	Criteria for earthquake Resistant Design of Structures

c.	IS code 875-1987: 	Code of Practice for Design Loads

Part 1 	Dead Loads

Part 2 	Imposed Loads

Part 5	Special loads and Load combination

d.	IS code 13920-1993: Ductile Detailing of RCC structures subjected to seismic force code of practice

e.	Nepal National Building Code (NBC 105): Seismic Design of Buildings in Nepal

f.	RCC structures by A.K. Jain

g.	RCC structures by S.N. Sinha

h.	Limit State Design by Vargees

i.	Structural Design of Multistoried building by U.H. Varyani

j.	IS code SP 16 Design guidelines for RCC Structures

k.	IS code SP 34 for detailing of RCC structures