STRUCTURAL ANALYSIS AND DESIGN OF

EARTHQUAKE RESISTANT BUILDING

STRUCTURALREPORT

Owner : MR. RAJENDRA ARYAL

Submitted to:

Sankharapur Municipality-9, Kathmandu

Prepared By:

Er. Menaka Suwal

DECEMBER 2025

- Table of Contents

1	INTRODUCTION	3

2	DESIGN PHILOSOPHY	3

3	DESIGN CODES AND STANDARDS	3

4	GENERAL DESIGN CRITERIA	5

4.1	MATERIALS	5

4.2	SOIL BEARING CAPACITY	5

4.3	METHODOLOGY	5

5	DESIGN LOAD	6

5.1	DEADLOAD	6

5.2	IMPOSED LOAD	7

5.3	EARTHQUAKE LOAD	8

6	LOAD COMBINATION	11

7	STRUCTURAL ANALYSIS USING ETABS	11

8	DESIGN METHODS OF STRUCTURAL ELEMENTS	12

9	LIMITSTATEMETHOD	12

11	DETAILING OF THE STRUCTURAL ELEMENTS:	14

12	DESIGN	36

12.1	COLUMN DESIGN	36

12.2	BEAM DESIGN	38

12.3	SLAB DESIGN	40

12.4	STAIR DESIGN	40

12.5	FOUNDATION DESIGN	43

13	ACCEPTABILITY CRITERIA	45

13.1       STORY DRIFT	45

14	DESIGN INPUT IN MUNICIPALITY FORMAT	51

## INTRODUCTION

The purpose of this report is to present general overview on structural analysis & design calculation of the two and half storey residential Building at Kathmandu owned by Mr. Rajendra Aryal. This report presents the criteria and methodology that have been used throughout in structural calculation which forms a basis for structural analysis and design regarding the assumption of loadings, follow up of the standards as well as the for relevant methodologies.

## DESIGN PHILOSOPHY

The past devastating earthquakes have proved the vulnerability of most of the vernacular buildings of Nepal. Enormous life and property were lost due to the collapse  of  buildings Earthquakes can neither be prevented  nor predicted  precisely.  But  large- scale destruction can be minimized by employing seismic-resistant  measures  in buildings. This can be achieved by the use of existing building materials in appropriate analysis.

The structures of buildings are designed to provide the spatial requirements in  accordance with the purpose of the buildings taking into account  the aesthetic  aspect and provision for various services  and systems  necessary for  the operation  of  airport. In order to fulfill such requirements, the structural materials  and the structural  system have been determined taking into account the availability  of materials,  cost efficiency and structural integrity, i.e. stability, strength and serviceability;  the three main  factors to  be in corporated  in the design  of all structures.

## DESIGN CODES AND STANDARDS

The structural design is based primarily on the current National Building Codes and Standards of India, while the relevant British Standards, European Standards or National Building Codes of Nepal are referred to appropriately for the areas/provisions that are   addressed in the Indian Standards or as required for the purpose of design. The main design standards followed for structural design are given below, indicating theirarea of application.

| S.N. | Codes and Standards | Description |
| --- | --- | --- |
| 1 | IS 456: 2000 | Plain and Reinforced Concrete - Code of Practice |
| 2 | IS 875 (Part 1): 1987 | Code of Practice for Design Loads (other than Earthquake) for Buildings and Structures: Part 1 Dead Loads – Unit Weights of Building Material and Stored Materials (Second Revision) |
| 3 | IS 875 (Part 2): 1987 | Code of Practice for Design Loads (other than Earthquake) for Buildings and Structures |
| 4 | IS 875 (Part 3): 1987 | Code of Practice for Design Loads (other than Earthquake) for Buildings and Structures |
| 5 | SP 34: 1987 | Handbook on Concrete Reinforcement and Detailing |
| 6 | IS 13920: 1993 | Ductile detailing of reinforced concrete structures subjected to seismic forces - Code of practice |
| 7 | IS 1893: 2016 | Criteria for Earthquake Resistant Design of structures |
| 8 | IS 383: 1970 | Specification for coarse and fine aggregates from natural sources |
| 9 | IS 1786: 1985 | Specification for high strength deformed steel bars and wires for concrete reinforcement (superseding IS:1139 - 1966) |
| 10 | IS 12269: 1987 | Specification for 53 Grade Ordinary Portland Cement. |
| 11 | IS 1904: 1986 | Design and Construction of Foundation in soils: General Requirements. |
| 12 | IS 800: 2007 | Code of Practice for General Construction in Steel (Third Revision) |
| 13 | NBC 104: 1994 | Wind Load |
| 14 | NBC 105: 2077 | Seismic Design of Buildings in Nepal |
| 15 | NBC 205: 2024 | Ready-To-Use Detailing Guideline for Low Rise Reinforced Concrete Buildings Without Masonry Infill |
| 16 | NBC 109: 1994 | Masonry: Unreinforced |
| 17 | NBC 202: 1994 | Mandatory Rules of Thumb Load Bearing Masonry |
| 18 | EN 1998 - 1 | Eurocode 8: Design of structures for earthquake resistance - Part 1: General rules, seismic actions and rules for buildings |

## GENERAL DESIGN CRITERIA

## MATERIALS

The materials to be used in structural design are as follows.

Concrete

Cast-in-situ concrete for all structural elements:

For Slabs, Beams	-	M20

Columns,	-	M25

For Footings, Lintel/Earthquake Bands	-	M20

Reinforcing Bars

High Strength deformed steel bars:  Grade Fe 500 to 1786 (14% minimum elongation)

## SOIL BEARING CAPACITY

In absence of soil test report, the presumed bearing capacity of foundation for the

site considering combined footing at 2 m depth is 120 KN/m2.

## METHODOLOGY

ETABS version 20 has been used for the linear static and dynamic analysis and design of three- dimensional structures, in which the spatial distribution of the mass and stiffness of the structure is adequate for the calculation of the significant features of structures. The structural elements of reinforced concrete were designed to Limit State Theory, while the structural steel elements have been designed to the Permissible Stress Theory.

The major structural elements were designed using design spreadsheets/software and verified manually too following the design aids SP16, IS 456 and IS  800 for reinforced  concrete  structures and structural  steel elements respectively.

Structures designed and built in seismic regions shall fulfill following fundamental requirements ie. Life safety and damage limitation. For the verification of performance requirements, following limit state shall be checked.

- Ultimate Limit State (ULS)

Ultimate limit states are associated with collapse or with other forms of structural failure which might endanger the safety of people. Design for ultimate limit state represents a procedure that ensures the probability of collapse of a structure is at an acceptable level.

- Serviceability Limit State(SLS)

Damage limitation states are associated with damage beyond which specified service requirements are no longer met. It represents a level of force within the structure below which there is a high degree of assurance that the structure can continue to be used as originally intended without repair.

Design loads for all structures have  been in  accordanc to  the criteria  described below.

## DESIGN LOAD

## DEAD LOAD

Dead Load on the structure comprise the self-weight of the member; weight of the finishes and partition walls. These are usually dependent upon the constructional features and have to be assumed in order to design various structural concrete members. The Loads on the beams due to the slabs were calculated according to clause 23.5 of IS 456 – 2000. The Wall Load is taken for thickness of either 230 mm or 115 mm as per Architectural Drawing and suitable reduction is made for Window and Door Opening.

All loads/forces due to gravity on the components of the building structure  including the  structures  self-weight,  roofing,  flooring,  suspended ceiling,  wall/partition, services including machinery, piping, rack with all associated finishing permanently attached  thereto  are calculated  in accordance with IS: 875 (Part 1) -1987.

As the software, we have used, generates the self-weight of the Structural member by itself, we  have not calculated the self-weight.

The structure has been designed considering following ideal loads:

Unit Weight (Material)

    - Density of Concrete	= 25kN/m3

    - Live Load	= 0.75 kN/m2 for Not accessible Floor

= 1.5 kN/m2 for Accessible Floor

= 3.00 kN/m2 for Staircase and Corridors

=2.0 kN/m2 for rooms

    - Floor Finishing Load	= 1.2kN/m2

    - Density of Brick Wall	=19.2kN/m2

    - Soil Bearing Capacity (adopted)= 120 kN/m2

| Self-weight | = | DL |
| --- | --- | --- |
| Wall Load | = | DL |
| Floor Finishing | = | DL |

Dead loads  were considered  separately as;

## IMPOSED LOAD

The load assumed to be produced by the intended use and occupancy of the building, including the loads of movable partition, impact, vibration, but excluding  wind, seismic, snow and other loads due to  temperature changes,  creep,  shrinkage,  differential settlement, etc., in accordance with IS 875  (Part2)  –  1987.  The words, “liveload”,here  in  or elsewhere shall be readas “imposedload”as synonymous.

| Wall Load | (Assuming 30 % opening) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Wall load calculation | B` | Density | H | Without opening | With opening | Unit |
| 9" wall(on grid) | 0.23 | 19.2 | 2.39 | 10.55 | 7.39 | KN/m |
| Plaster 12.5mm | 0.013 | 20.4 | 2.39 | 0.61 | 0.43 | KN/m |
| Total |  |  |  | 11.16 | 7.81 | KN/m |
| Adopted value |  |  |  | 11.5 | 8 | KN/m |
| 4" wall (on grid) | 0.115 | 19.2 | 2.39 | 5.28 | 3.96 | KN/m |
| Plaster 12.5mm | 0.013 | 20.4 | 2.39 | 0.61 | 0.43 | KN/m |
| Total |  |  |  | 5.89 | 4.38 | KN/m |
| Adopted value |  |  |  | 6 | 4.5 | KN/m |
| Parapet wall |  |  |  |  |  |  |
| 4" wall (on grid) | 0.115 | 19.2 | 1.07 | 2.36 |  | KN/m |
| Plaster 12.5mm | 0.013 | 20.4 | 1.07 | 0.27 |  | KN/m |
| Total |  |  |  | 2.63 |  | KN/m |
| Adopted value |  |  |  | 2.7 |  | KN/m |

## EARTHQUAKELOAD

Response Spectrum including modal analysis  hasbeen  used as per  theNBC 105:2020  tocalculate  theearthquake  forces  on the structure.  Base  shear  was  derivedfrom the  horizontal seismic base shear (Vb)  given by:

V=Cd(T1)W	(NBC 105:2020, Cl. 6.2(1))

Where;

W=	Seismic  weightof  thestructure  equalto  thetotal  dead  loadplusappropriate amounts  of specified imposedload

Cd(T1) = Horizontal base shear coefficient given by:

- Ultimate Limit State

(NBC 105:2020,  Cl.6.2(1))

Where,

Elastic Site Spectra(C(T1)) is given by

(NBC 105:2020,  Cl.4.1.1)

Where,

Ch(T )= 2.25 Spectral Shape Factor 	 (NBC 105:2020,  Cl.4.1.2)

I = 1.0Importantfactor(NBC 105:2020,  Cl.4.1.5)

Z = 0.35Seismic Zoning Factor(KATHMANDU)	(NBC 105:2020,  Cl.4.1.4)

So,C(T)=2.25*0.35*1.0

=0.7875

R= The ductility factor=4                           (NBC 105:2020,  Cl.5.3.1 table 5-2)

u=The overstrength factor=1.5                  (NBC 105:2020,  Cl.5.4.1 table 5-2)

Cd(T1)	= 0.7875/(4*1.5)

= 0.1313

- Serviceability Limit State

(NBC 105:2020,  Cl.6.2(2))

Where,

Cs(T )= 0.2C(T)Elastic Site Spectra	 (NBC 105:2020,  Cl.4.1.2)

= 0.2*0.7875

= 0.1575

s= The overstrength factor=1.25                     (NBC 105:2020,  Cl.5.4.2 table 5-2)

So,

Cd(T 1)=0.1575/1.25

=0.126

Period of vibration

The fundamental translation period shall be determined by the following two methods.

- Rayleigh Method

- Emperical Equations

The time period obtained by empirical equations shall be compared with that obtained by Rayleigh Method and the lesser value of the two shall be adopted for the design action.

Emperical Equations

The  approximate   fundamental   natural   period   of  vibration   (Ta),  inseconds,   of  a

moment resisting frame building without brick infill panels may be estimated by the empirical expression:

Ta =  0.075  h0.75 for RCMRF building

=   0.085 h0.75  for steel MRF building

Where:

h  =Height  of  building from foundation,  in  m.

T1=  0.075* 8.35440.75

=0.375sec.

The approximate fundamental time period as per NBC 105: 2020 CL: 5.1.3 (Page 29)

T1=  1.25*0.53

= 0.468 sec

Rayleigh Method

The seismic weight at each level, Wi, shall be taken as the sum of the dead loads and the factored seismic live loads between the mid-heights of adjacent stories.

The seismic live load shall be determined as given in Table 5-1.

## LOAD COMBINATION

For design of reinforced concrete elements, the following load combinations were considered according to NBC 105:2020 CL 3.6.1:

- 1.2DL+1.5LL

- DL+0.3LL+SLSEQX

- DL+0.3LL-SLSEQX

- DL+0.3LL+SLSEQY

- DL+0.3LL-SLSEQY

- DL+0.3LL+ULSEQX

- DL+0.3LL-ULSEQX

- DL+0.3LL+ULSEQY

- DL+0.3LL-ULSEQY

Where:

DL=DeadLoad

LL= LiveLoad

ULSEQX= Ultimate limit state Earthquake load inX-direction

ULSEQY= Ultimate limit state Earthquake load inY-direction

SLSEQX= Serviceability limit state Earthquake load inX-direction

SLSEQY= Serviceability limit state Earthquake load inY-direction

## STRUCTURAL ANALYSIS USING ETABS

The building was modeled in structural analysis program ETABS Ver. 20 using frame elements. Slabs were modeled using shell elements to distribute loads on frame elements.3-D Modeling was prepared using this FEM modelingbased software considering beams and columns as FRAME element, and Slab asshell element wherever possible.

Equivalent Static Method was carried out for its analysis in lateral loading due toearthquake

## DESIGN METHODS OF STRUCTURAL ELEMENTS

We have followed Indian Standard Codeof  Practicefor  Plainand  Reinforced Concrete IS: 456-2000 for design of Structural Elements. This incorporates the two methods ofStructural Design of RC structures specified as:

- Working Stress Method based on the Working loads in conjunction with permissible stresses in thematerials.

- Limit State Method based on safety and serviceability requirements associated with the design loads and design strengths of the materials. These design loads and design strengths are obtained by applying partial safety factors for characteristic loads and strengths of the materials concrete andsteel.

We have followed the limit state method which is incorporated in IS:456-2000. It is consistent with the new philosophy of design termed limit state approach which was incorporated in the Russian Code – 1954, the British code BS 8110 – 1985 and the American  Code ACI 318 – 1989.

## LIMIT STATE METHOD

- LimitStates

The Limit State method of design covers the various forms  of  failure.  There are several limit state at which the structure  ceasesto  function,  themost  importantamong  thembeing,

- The limit state of collapse or total failure ofstructure.

It  correspondsto  themaximum  loadcarrying  capacity.  Violation  ofcollapse  implies failure. This limit state corresponds to Flexure, Compression,  Shear  and Torsion.

- The limit state of serviceability which includes excessive deflection and  excessive local damage. Excessive deflection  adverselyaffects  thefinishes  and excessive local damage results in cracking of concrete, which impairs the efficiency, or appearance of the structure.

- DesignEquationsinLimitStateMethod:

- Singly ReinforcedSections:

The depth of Neutral Axis xu is obtained as Xu	=	[ ( 0.87Astfy / 0.36 fck  b)]

For under Reinforced Sections, the neutral Axis depth Xu is limited to a value of  Xu

max < 0.48 d   for steel of fy = 415 N /mm2

If the neutral axis depth is less than these limiting values, the ultimate moment is computed

By the equation:

Mu=	0.87 fy Astd [ 1 – { fyAst/ b d fck  }]

If the Neutral Depth is equal to the limiting values, then the section is balanced or  limiting  and the moment capacity is evaluated by theequation.

Mu,lim=	0.36 X u, max / d [ 1 – 0.42 X u,max/ d ] b d 2 fck

Mu,lim=	0.138  fckb d 2  ( for HYSD bars with fy  = 415 N /mm2  )  Area of steelina balanced section is givenby:

Ast=	[ 0.36 f ck b X u,max/ 0.87 f y]

- Doubly ReinforcedSections:

The ultimatemoment resistance of sections with compressionreinforcement (doubly   reinforced Section) is computed by the equation.

[ M u  – Mu,lim]	=	f scAsc( d – d ’)

| Where, | M u | = | Moment of Resistance of the doubly reinforced sections |
| --- | --- | --- | --- |
|  | M u,lim | = | Limiting moment of Resistance of a section without |
|  |  |  | Compression reinforcement |
|  | d d’ | = = | Effective Depth Depth of Compression Reinforcement from compression face |
|  | fsc | = | Design stress in compression reinforcement |

The total Area of Tension Reinforcement is obtained asAst=	Ast1 +Ast2Where,

| Ast1 | = | Total Area of Tensile Reinforcement |  |
| --- | --- | --- | --- |
| Ast2 | = | Area of Tensile Reinforcement for a singly Reinforced | Section for Mu,kim |
| Asc | = | Ascfsc/ 0.87 fy |  |

The nominal shear stress Tv in beams of uniform depth is obtained as

| T v | = | Vu / b d |
| --- | --- | --- |
| Where, Vu B d | = = = | Ultimate shear Force due to design loads Breadth of member effective depth |

The permissible shear stress Tc in beam depends upon the percentage of longitudinal reinforcement. Where Tc < Tv, minimum shear Reinforcement is  given by

Asv / bSv> = 0.4 / fy

If Tv exceeds Tc, but  isless  thanTc  max,  then  Shear Reinforcements  aredesigned  to  take up the balanced shear Vu – Tcbd

The Strength of Shear Reinforcement Vus is calculated as  Vus      =0.87 fi Asv d / Sv

- ANALYSIS AND DESIGN OF THE STRUCTURALELEMENTS:

The Structure is analyzed and designed by standard software ETABS Vr.20. It is the World’s most popular and  widelyused  structuralEngineering  Software.  It  isequipped  with the powerful analysis, design, graphics, and visualization  capabilities.

General Information on Structural Elements of the Building

| Element | Description | Grade of Concrete | Remarks |
| --- | --- | --- | --- |
| Column | 350 mm X 350 mm | M20 |  |
| Main Beam | 350 mm X 230 mm | M20 |  |
| Slab | 125 mm | M20 |  |
| Waist Slab | 125 mm | M20 |  |
| Foundation | Isolated, Combined and Strap Footing | M25 | Soil Bearing Capacity is assumed to be 120kN/m2 |

## DETAILING OF THE STRUCTURALELEMENTS:

The Reinforcement detailing of most of  theimportant  structuralcomponents  have  been shown in drawing. They confirm with the relevant sections of the IS Codes IS  456-2000, IS 1893-1984, SP-16, and SP –34–1987.

Frame Structure and Element

Wall Dead Load Frames (KN/m)

Live load (KN/m2)

longitudinal Reinforcements (X - Z Axis) (mm2)

Grid 1-1

Grid 2-2

Grid 3-3

Grid 3-3

Grid 3-3

Grid 3-3

`

Beam Reinforcements (X -Y Axis) First Floor Beam

Beam Reinforcements (X -Y Axis) Second Floor Beam

Beam Reinforcements (X -Y Axis) Third Floor Beam

Bending Moment Diagram

Axial Force Diagram

Shear Force Diagram

Seismic Load

Horizontal Base shear coefficient

- Ultimate Limit State

Cd(T1)	= 0.7875/(4*1.5)

= 0.1313

- Serviceability Limit State

Cd(T 1)= 0.1575/1.25

=0.126

Seismic Weight of the Structure=3871.07KN

Horizontal Seismic Base shear

V= Cd(T1)*W

Where Cd(T1)=Horizontal Base shear coefficient

W=Seismic Weight of the Structure

- Ultimate Limit State Base Shear

V=0.1313*3871.07

=508.27KN

- Serviceability Limit State Base Shear

V=0.126*3871.07

=487.75KN

TABLE:  Auto Sesmic Load

| TABLE: Load Pattern Definitions - Auto Seismic - User Coefficient |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Is Auto Load | Ecc Ratio | Top Story | Bottom Story | C | K | Weight Used | Base Shear |  |
|  |  |  |  |  |  |  | kN | kN |  |
| EQx SLS | No | 0.1 | Story3 | Base | 0.126 | 1.0655 |  |  |  |
| EQx SLS(1/6) | Yes | 0.1 | Story3 | Base | 0.126 | 1.0655 | 2368.6491 |  | 298.4498 |
| EQx SLS(2/6) | Yes | 0.1 | Story3 | Base | 0.126 | 1.0655 | 2368.6491 |  | 298.4498 |
| EQx SLS(3/6) | Yes | 0.1 | Story3 | Base | 0.126 | 1.0655 | 2368.6491 |  | 298.4498 |
| EQx SLS(4/6) | Yes | 0.1 | Story3 | Base | 0.126 | 1.0655 | 2368.6491 |  | 298.4498 |
| EQx SLS(5/6) | Yes | 0.1 | Story3 | Base | 0.126 | 1.0655 | 2368.6491 |  | 298.4498 |
| EQx SLS(6/6) | Yes | 0.1 | Story3 | Base | 0.126 | 1.0655 | 2368.6491 |  | 298.4498 |
| EQx ULS | No | 0.1 | Story3 | Base | 0.131 | 1.0655 |  |  |  |
| EQx ULS(1/3) | Yes | 0.1 | Story3 | Base | 0.131 | 1.0655 | 2368.6491 |  | 310.293 |
| EQx ULS(2/3) | Yes | 0.1 | Story3 | Base | 0.131 | 1.0655 | 2368.6491 |  | 310.293 |
| EQx ULS(3/3) | Yes | 0.1 | Story3 | Base | 0.131 | 1.0655 | 2368.6491 |  | 310.293 |
| EQy SLS | No | 0.1 | Story3 | Base | 0.126 | 1.0655 |  |  |  |
| EQy SLS(1/3) | Yes | 0.1 | Story3 | Base | 0.126 | 1.0655 | 2368.6491 |  | 298.4498 |
| EQy SLS(2/3) | Yes | 0.1 | Story3 | Base | 0.126 | 1.0655 | 2368.6491 |  | 298.4498 |
| EQy SLS(3/3) | Yes | 0.1 | Story3 | Base | 0.126 | 1.0655 | 2368.6491 |  | 298.4498 |

| TABLE: Modal Participating Mass Ratios |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case | Mode | Period | UX | UY | UZ | SumUX | SumUY | SumUZ |
|  |  | sec |  |  |  |  |  |  |
| Modal | 1 | 0.605 | 0.004 | 0.8389 | 0 | 0.004 | 0.8389 | 0 |
| Modal | 2 | 0.556 | 0.8358 | 0.0046 | 0 | 0.8398 | 0.8436 | 0 |
| Modal | 3 | 0.518 | 0.0176 | 0.0007 | 0 | 0.8574 | 0.8443 | 0 |
| Modal | 4 | 0.192 | 0.0001 | 0.1145 | 0 | 0.8575 | 0.9588 | 0 |
| Modal | 5 | 0.185 | 0.0836 | 0.0013 | 0 | 0.941 | 0.9601 | 0 |
| Modal | 6 | 0.169 | 0.0254 | 0.0017 | 0 | 0.9665 | 0.9618 | 0 |
| Modal | 7 | 0.112 | 0.0119 | 0.0205 | 0 | 0.9783 | 0.9823 | 0 |
| Modal | 8 | 0.111 | 0.0168 | 0.0174 | 0 | 0.9951 | 0.9996 | 0 |
| Modal | 9 | 0.101 | 0.0049 | 0.0004 | 0 | 1 | 1 | 0 |
| Modal | 10 | 0.018 | 0 | 0 | 0 | 1 | 1 | 0 |
| Modal | 11 | 0.018 | 0 | 0 | 0 | 1 | 1 | 0 |
| Modal | 12 | 0.017 | 0 | 0 | 0 | 1 | 1 | 0 |

## DESIGN

## COLUMN DESIGN

Ground Floor= B2

ETABS Concrete Frame Design

IS 456:2000 + IS 13920:2016  Column Section Design   (Summary)

Column Element Details

| Level | Element | Unique Name | Section ID | Combo ID | Station Loc | Length (mm) | LLRF | Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Story1 | C5 | 66 | Col 14"*14" | DConS19 | 0 | 2844.8 | 1 | Ductile Frame |

Section Properties

| b (mm) | h (mm) | dc (mm) | Cover (Torsion) (mm) |
| --- | --- | --- | --- |
| 355.6 | 355.6 | 60 | 30 |

Material Properties

| Ec (MPa) | fck (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 22360.68 | 20 | 1 | 500 | 500 |

Design Code Parameters

| ɣC | ɣS |
| --- | --- |
| 1.5 | 1.15 |

Axial Force and Biaxial Moment Design For Pu , Mu2 , Mu3

| Design Pu kN | Design Mu2 kN-m | Design Mu3 kN-m | Minimum M2 kN-m | Minimum M3 kN-m | Rebar Area mm² | Rebar % % |
| --- | --- | --- | --- | --- | --- | --- |
| 16.5303 | -2.28 | 88.0464 | 0.3306 | 0.3306 | 1818 | 1.44 |

Axial Force and Biaxial Moment Factors

|  | K Factor Unitless | Length mm | Initial Moment kN-m | Additional Moment kN-m | Minimum Moment kN-m |
| --- | --- | --- | --- | --- | --- |
| Major Bend(M3) | 0.618627 | 2489.2 | 35.7968 | 0 | 0.3306 |
| Minor Bend(M2) | 0.669084 | 2489.2 | 1.0935 | 0 | 0.3306 |

Shear Design for Vu2 , Vu3

|  | Shear Vu kN | Shear Vc kN | Shear Vs kN | Shear Vp kN | Rebar Asv /s mm²/m |
| --- | --- | --- | --- | --- | --- |
| Major, Vu2 | 52.4763 | 62.971 | 42.0464 | 35.7041 | 394.16 |
| Minor, Vu3 | 19.6676 | 62.971 | 42.0464 | 19.6676 | 394.16 |

Joint Shear Check/Design

|  | Joint Shear Force kN | Shear VTop kN | Shear Vu,Tot kN | Shear Vc kN | Joint Area cm² | Shear Ratio Unitless |
| --- | --- | --- | --- | --- | --- | --- |
| Major Shear, Vu2 | N/N | N/N | N/N | N/N | N/N | N/N |
| Minor Shear, Vu3 | N/N | N/N | N/N | N/N | N/N | N/N |

(1.4) Beam/Column Capacity Ratio

| Major Ratio | Minor Ratio |
| --- | --- |
| N/N | N/N |

Additional Moment Reduction Factor k (IS 39.7.1.1)

| Ag cm² | Asc cm² | Puz kN | Pb kN | Pu kN | k Unitless |
| --- | --- | --- | --- | --- | --- |
| 1264.5 | 18.2 | 1819.8075 | 448.6825 | 16.5303 | 1 |

Additional Moment (IS 39.7.1)

|  | Consider Ma | Length Factor | Section Depth (mm) | KL/Depth Ratio | KL/Depth Limit | KL/Depth Exceeded | Ma Moment (kN-m) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Major Bending (M3 ) | Yes | 0.875 | 355.6 | 4.33 | 12 | No | 0 |
| Minor Bending (M2 ) | Yes | 0.875 | 355.6 | 4.684 | 12 | No | 0 |

Notes:

N/A: Not Applicable

N/C: Not Calculated

N/N: Not Needed

Column Design Summary

| SN | Column Name | Column Size |  | Floor | Reinforcement |  |  | Lateral Ties |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L (in) | B (in) |  | AR(mm2) | Bars | AP(mm2) | Dia./Leg | Side | Mid |
| 1 | A1 | 14 | 14 | GF | 1403 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 1F | 1012 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 2F | 1356 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 3F |  |  |  |  |  |  |
|  |  |  |  | 4F |  |  |  |  |  |  |
| 2 | A2 | 14 | 14 | GF | 1595 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 1F | 1081 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 2F | 1128 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 3F |  |  |  |  |  |  |
|  |  |  |  | 4F |  |  |  |  |  |  |
| 3 | A3 | 14 | 14 | GF | 1446 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 1F | 1012 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 2F | 1315 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 3F |  |  |  |  |  |  |
|  |  |  |  | 4F |  |  |  |  |  |  |
| 4 | B2 | 14 | 14 | GF | 1818 | 4$20+4$16 | 2061.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 1F | 1212 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 2F | 1123 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 3F |  |  |  |  |  |  |
|  |  |  |  | 4F |  |  |  |  |  |  |
| 5 | B3 | 14 | 14 | GF | 1783 | 4$20+4$16 | 2061.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 1F | 1133 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 2F | 1388 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 3F |  |  |  |  |  |  |
|  |  |  |  | 4F |  |  |  |  |  |  |
| 6 | C1 | 14 | 14 | GF | 1450 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 1F | 1012 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 2F | 1135 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 3F |  |  |  |  |  |  |
|  |  |  |  | 4F |  |  |  |  |  |  |
| 7 | C2 | 14 | 14 | GF | 1784 | 4$20+4$16 | 2061.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 1F | 1383 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 2F | 1133 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 3F |  |  |  |  |  |  |
|  |  |  |  | 4F |  |  |  |  |  |  |
| 8 | C3 | 14 | 14 | GF | 1728 | 4$20+4$16 | 2061.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 1F | 1122 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 2F | 1119 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 3F |  |  |  |  |  |  |
|  |  |  |  | 4F |  |  |  |  |  |  |
| 9 | D1 | 14 | 14 | GF | 1396 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 1F | 1609 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 2F |  |  |  |  |  |  |
|  |  |  |  | 3F |  |  |  |  |  |  |
|  |  |  |  | 4F |  |  |  |  |  |  |
| 10 | D2 | 14 | 14 | GF | 1406 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 1F | 1020 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 2F | 1354 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 3F |  |  |  |  |  |  |
|  |  |  |  | 4F |  |  |  |  |  |  |
| 11 | D3 | 14 | 14 | GF | 1133 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 1F | 1012 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 2F | 1591 | 8$16 | 1608.00 | 8/4L | 4" c/c | 6" c/c |
|  |  |  |  | 3F |  |  |  |  |  |  |
|  |  |  |  | 4F |  |  |  |  |  |  |

## 12.2 BEAMDESIGN

Ground Floor = C2-C3

| ETABS Concrete Frame Design IS 456:2000 + IS 13920:2016 Column Section Design (Summary) Column Element Details Level; Element; Unique Name; Section ID; Combo ID; Station Loc; Length (mm); LLRF; Type. Story1; C5; 66; Col 14"*14"; DConS19; 0; 2844.8; 1; Ductile Frame. Section Properties b (mm); h (mm); dc (mm); Cover (Torsion) (mm). 355.6; 355.6; 60; 30. Material Properties Ec (MPa); fck (MPa); Lt.Wt Factor (Unitless); fy (MPa); fys (MPa). 22360.68; 20; 1; 500; 500. Design Code Parameters ɣC; ɣS. 1.5; 1.15. Axial Force and Biaxial Moment Design For Pu , Mu2 , Mu3 Design Pu kN; Design Mu2 kN-m; Design Mu3 kN-m; Minimum M2 kN-m; Minimum M3 kN-m; Rebar Area mm²; Rebar % %. 16.5303; -2.28; 88.0464; 0.3306; 0.3306; 1818; 1.44. Axial Force and Biaxial Moment Factors K Factor Unitless; Length mm; Initial Moment kN-m; Additional Moment kN-m; Minimum Moment kN-m. Major Bend(M3); 0.618627; 2489.2; 35.7968; 0; 0.3306. Minor Bend(M2); 0.669084; 2489.2; 1.0935; 0; 0.3306. Shear Design for Vu2 , Vu3 Shear Vu kN; Shear Vc kN; Shear Vs kN; Shear Vp kN; Rebar Asv /s mm²/m. Major, Vu2; 52.4763; 62.971; 42.0464; 35.7041; 394.16. Minor, Vu3; 19.6676; 62.971; 42.0464; 19.6676; 394.16. Joint Shear Check/Design Joint Shear Force kN; Shear VTop kN; Shear Vu,Tot kN; Shear Vc kN; Joint Area cm²; Shear Ratio Unitless. Major Shear, Vu2; N/N; N/N; N/N; N/N; N/N; N/N. Minor Shear, Vu3; N/N; N/N; N/N; N/N; N/N; N/N. (1.4) Beam/Column Capacity Ratio Major Ratio; Minor Ratio. N/N; N/N. Additional Moment Reduction Factor k (IS 39.7.1.1) Ag cm²; Asc cm²; Puz kN; Pb kN; Pu kN; k Unitless. 1264.5; 18.2; 1819.8075; 448.6825; 16.5303; 1. Additional Moment (IS 39.7.1) Consider Ma; Length Factor; Section Depth (mm); KL/Depth Ratio; KL/Depth Limit; KL/Depth Exceeded; Ma Moment (kN-m). Major Bending (M3 ); Yes; 0.875; 355.6; 4.33; 12; No; 0. Minor Bending (M2 ); Yes; 0.875; 355.6; 4.684; 12; No; 0. Notes: N/A: Not Applicable N/C: Not Calculated N/N: Not Needed Beam Reinforcement Detail |
| --- |
| Floor | Size | Main Reinforcement Through |  | Extra Reinforcement Ends |  | Stirrup |  |
|  |  | Top | Bottom | Top | Bottom | End | Middle |
| PlinthTie | 12"x12" | 2-16φ+1-12 φ | 3-16φ |  |  | 8φ4"c-c | 8φ6"c-c |
| Ground Floor | 9"x14" | 2-16φ+1-12 φ | 3-16φ |  |  | 8φ4"c-c | 8φ6"c-c |
| 1st Floor | 9"x14" | 2-16φ+1-12 φ | 3-16φ |  |  | 8φ4"c-c | 8φ6"c-c |
| Top Floor | 9"x14" | 2-16φ+1-12 φ | 3-16φ |  |  | 8φ4"c-c | 8φ6"c-c |

## 12.3 SLAB DESIGN

| Design of Two way Slab by Limit State Method |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| (As per IS 456:2000) |  |  |  |  |  |  |
| 1 | Data |  |  |  |  |  |
|  | Longer span of slab, ly | 3.531 | m |  |  |  |
|  | Shorter span of slab, lx | 3.506 | m |  |  |  |
|  | Materials used |  |  |  |  |  |
|  | Concrete | M20 | 20 | N/mm2 |  |  |
|  | Steel | Fe500 | 500 | N/mm2 |  |  |
| 2 | Effective span |  |  |  |  |  |
|  | Longer span of slab, ly | 3.354 | m |  |  |  |
|  | Shorter span of slab, lx | 3.329 | m |  |  |  |
|  | ly/lx | 1.008 |  |  |  |  |
|  | type of slab | <2 two way slab |  |  |  |  |
|  | Support Condition : |  |  |  |  |  |
|  | Two Adjacent Edge Discontinuous |  |  |  |  |  |
| 3 | Calculation of loads |  |  |  |  |  |
|  | i) Dead load |  |  |  |  |  |
| i | Slab thickness | 125 | mm |  |  |  |
|  | Self weight of concrete | 25 | KN/m3 |  |  |  |
|  | Dead load of slab = 25 * 0.125 | 3.125 | KN/m2 |  |  |  |
| ii | Screeding thickness | 30 | mm |  |  |  |
|  | Self weight of screeding | 20.4 | KN/m3 |  |  |  |
|  | Dead load of screeding = 20.4 * 0.03 | 0.612 | KN/m2 |  |  |  |
| iii | marble flooring thickness | 19 | mm |  |  |  |
|  | Self weight of marble | 27 | KN/m3 |  |  |  |
|  | Dead load of marble = 27 * 0.019 | 0.513 | KN/m2 |  |  |  |
| iv | Cement plaster thickness | 12.5 | mm |  |  |  |
|  | Self weight of plaster | 20.4 | KN/m3 |  |  |  |
|  | Dead load of plaster = 20.4 * 0.0125 | 0.255 | KN/m2 |  |  |  |
|  | Total dead load on slab | 4.505 | KN/m2 |  |  |  |
|  | ii) Live load |  |  |  |  |  |
|  | For Residential Building | 2 | KN/m2 |  |  |  |
|  | Total design load (DL+LL) | 6.505 | KN/m2 |  |  |  |
|  | Factored load = 1.5 * design load | 9.7575 | KN/m2 |  |  |  |
| 4 | Calculation of moments |  |  |  |  |  |
|  | As per code, | Shorter Direction |  | Longer Direction |  |  |
|  | -ve moment at support | Support, αx1 | 0.047 | Support, αy1 | 0.047 |  |
|  | +ve moment at mid | mid, αx2 | 0.035 | mid, αy2 | 0.035 |  |
|  | Along shorter span, |  |  |  |  |  |
|  | Mx = αx1*wu*lx | 6.407 | KNm |  |  |  |
|  | My = αy1*wu*lx | 6.346 | KNm |  |  |  |
|  | Along longer span, |  |  |  |  |  |
|  | Mx = αx2*wu*lx | 4.777 | KNm |  |  |  |
|  | My = αy2*wu*lx | 4.726 | KNm |  |  |  |
| 5 | Depth Calculation |  |  |  |  |  |
|  | Minimum depth of slab is calculated considering maximum bending moment |  |  |  |  |  |
|  | Maxm bending moment | 6.407 |  |  |  |  |
|  | Depth required, |  |  |  |  |  |
|  | d=sqrt(Mu/(0.36*xumax/d)*(1-0.42*xumax/d)*b*fck) |  |  |  |  |  |
|  | overall depth, D=d+φ/2+clear cover) |  | mm |  |  |  |
|  | For safe design, adopting depth of slab | 125 | mm |  |  |  |
|  | Effective depth | 96 | mm |  |  |  |
| 6 | Reinforcement Calculation | Main Bar | Dist. Bar |  |  |  |
| provide the reinforcement of dia |  | 10 | 8 | mm |  |  |
| S. No. | Description | Shorter Direction |  | Longer Direction |  |  |
|  |  | Support | mid | Support | mid |  |
| 1 | αx & αy | 0.047 | 0.035 | 0.047 | 0.035 |  |
| 2 | BM | 6.41 | 4.78 | 6.35 | 4.73 |  |
| 3 | Ast required | 145.30 | 107.32 | 143.87 | 106.15 |  |
| 4 | Ast min | 150.00 | 150.00 | 150.00 | 150.00 |  |
| 5 | Spacing reqd | 523.81 | 523.81 | 335.24 | 335.24 |  |
| 6 | Spacing prov. | 150.00 | 150.00 | 150.00 | 150.00 |  |
| 7 | Ast provided | 523.81 | 523.81 | 335.24 | 335.24 |  |
| 8 | % of steel provided | 0.42 | 0.42 | 0.27 | 0.27 |  |
| 7 | Check |  |  |  |  |  |
| 7.1 | Minimum reinforcement |  |  |  |  |  |
|  | Min Ast = 0.12% bD | 150 | mm2 | < Ast provided in all sections |  |  |
| 7.2 | Check for deflection |  |  |  |  |  |
|  | From deflection control criteria |  |  |  |  |  |
|  | lx/d < (l/d)basic*αβγ | where |  |  |  |  |
|  | lx/d | 31.7 |  |  |  |  |
|  | α = modification factor for tension reinforcement |  |  |  |  |  |
|  | β = modification factor for compression reinf. |  | 1 |  |  |  |
|  | γ = reduction factor for ratios of span to eff. depth in flanged beam |  | 1 |  |  |  |
|  | (l/d)basic = | 23 |  |  | of IS 456 |  |
|  | For α, |  |  |  |  |  |
|  | fs = 0.58*fy*(Ast required/Ast provided) | 83.045 | N/mm2 |  |  |  |
|  | % of tensional reinforcement | 0.419 |  |  |  |  |
|  | α | 2 |  |  |  |  |
|  | now, (l/d)basic*αβγ | 46 |  | OK Safe |  |  |
|  | Since, lx/d < (l/d)basic*αβγ | SAFE |  |  |  |  |
| 7.3 | Check for Shear |  |  |  |  |  |
|  | Shear is checked along short span. |  |  |  |  |  |
|  | Maximum shear force, Vumax = w*lx/2 | 20.283 | KN |  |  |  |
|  | Normal shear stress, τv = V/bd | 0.193 | N/mm2 |  |  |  |
|  | % of steel provided = 100*Ast provided/bD | 0.249 |  |  |  |  |
|  | Shear capacity of section without reinforcement |  |  |  |  |  |
|  | From table 19, |  |  |  |  |  |
|  | τc | 0.360 | N/mm2 |  |  |  |
|  | Shear strength, τ'c = k*τc |  |  |  |  |  |
|  | For slab thickness 125mm, k | 1.3 |  |  |  |  |
|  | Shear strength, τ'c | 0.467 | N/mm2 |  |  |  |
|  | Since, τ'c > τv | OK Safe |  |  |  |  |
| 8 | Results |  |  |  |  |  |
|  | Overall depth of slab = | 125 | mm |  |  |  |
|  | Main bar along short span at support | 10mm ø @ 150mm c/c |  |  |  |  |
|  | Main bar along short span at mid | 10mm ø @ 150mm c/c |  |  |  |  |
|  | Distribution Bar along long span at support | 8mm ø @ 150mm c/c |  |  |  |  |
|  | Distribution bar along long span at mid | 8mm ø @ 150mm c/c |  |  |  |  |

| 12.4. Design of Staircase |  |  |
| --- | --- | --- |
| Y = | 25 | N/m3 |
| Effective Span | 3.3 | m |
| Riser (R) | 152 | mm |
| Tread (T) | 228 | mm |
| √ (R2+T2) | 310.16 | mm |
| Assume waist slab thickness (d) | 100 | mm |
| (D) | 125 | mm |
| Assumed grade of concrete (fck) | M20 |  |
| Assumed steel (fy) | Fe 500 |  |
| Loads on stairs |  |  |
| Self weight of waist slab | 3 82 | KN/m2 |
| Self weight o steps | 2.225 | KN/ m2 |
| Finish | 1 | KN/m2 |
| Live Load | 3 | KN/m2 |
| Total Load | 10.04 | KN/m2 |
| Loads on landing |  |  |
| Self weight of slab | 3.125 | KN/m2 |
| Finish | 1 | KN/m2 |
| Live Load | 3 | KN/m2 |
| Total Load | 7.125 | KN/m2 |
| Design Moment considering 1 m wide strip fo waist slab. |  |  |
| Reactions |  |  |
| RA = | 18.09 | KN |
| RB = | 12.92 | KN |
| Max moment at x= | 1.580 | m from A support |
| BMmax | 11.70 | KNm/m |
| Depth required |  |  |
| M=2.66bd^2 (for M20 and Fe500) |  |  |
| d | 66.32 | mm |
| d (provided) | 120 | mm (OK) |
| Ast required |  |  |
| BM= 0.87fyAst (d-(Ast*fy)/(fck*b)) |  |  |
| Ast | 235.71 | mm2 |
| φ = | 12 | mm |
| spacing | 480.00 | mm |
| Distributors |  |  |
| = 0.12% of b D | 150 | mm2 |
| φ = | 8 | mm |
| spacing | 335.24 | mm |
| Shear check |  |  |
| Max shear at distance d from the support |  |  |
| Vu = | 17.87 | KN |
| Τv = Vu/b d | 0.179 | N/mm |
| Τc |  |  |
| % = 100 * (Ast/b d) | 0.235713066 | % |
| Τc = | 0.44 | for M 20 |
| k = | 1.3 |  |
| k Τc = | 0.57 | N/mm2>Τv (OK) |
| Provide 12 mm φ @ 150 mm spacing c/c | Main bars |  |
| Provide 10 mm φ @ 150 mm spacing c/c | Distribution bars |  |

## FOUNDATION DESIGN

## 12.5. Design of foundation

The purpose of the foundation is to effectively support the superstructure by transmitting the applied load effects to the soil below, without exceeding the safe bearing capacity of the superstructure by ensuring settlement of the structure is within tolerable limits, and as nearly uniform as possible.

The choice of the type of the foundation depends not only on the type of the superstructure and the magnitude and types of reactions induced at the base of the superstructure, but also on the nature of the soil strata on top of which the substructure is to be founded.

The foundation used for this building is isolated foundation. Bearing Capacity of Soil is 150KN/m2.

| Showing Joint Reaction along respective joint: |  |  |
| --- | --- | --- |

| Isolated Footing Design |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Foundation Type : |  | F2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Required Data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| size of colunm= |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | l = |  | 0.35 |  | m |  |  |  |  |  |  |  |  |  |
|  | b = |  | 0.35 |  | m |  |  |  |  |  |  |  |  |  |
| Bearing Capacity of Soil= |  |  | 120 |  | KN/m² |  |  |  |  |  |  |  |  |  |
| Strength of Steel (fy)= |  |  | 500 |  | N/mm² |  |  |  |  |  |  |  |  |  |
| Strength of Concrete (fck)= |  |  | 20 |  | N/mm² |  | ( M 20 ) |  |  |  |  |  |  |  |
| Factor Load= |  |  | 337.608 |  |  |  |  |  |  |  |  |  |  |  |
| Axial Load = |  |  | 225.07 |  | KN |  |  |  |  |  |  |  |  |  |
| Approximate area of footing= |  |  | 1.88 |  | m² |  |  |  |  |  |  |  |  |  |
| Weight of footing including earth= |  |  | 56.27 |  |  |  |  |  |  |  |  |  |  |  |
| Axial Load = |  |  | 225.07 |  | KN |  |  |  |  |  |  |  |  |  |
| Total weight on soil= |  |  | 281.34 |  | KN |  |  |  |  |  |  |  |  |  |
| Actual Area= |  |  | 2.34 |  | m² |  |  |  |  |  |  |  |  |  |
| Size of Square Footing = |  |  | 1.531 |  |  |  | 5.022257341 |  |  |  |  |  |  |  |
| Adopted size= |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | L = |  | 1.83 |  | m |  | 6.00 |  |  |  |  |  |  |  |
|  | B = |  | 1.83 |  | m |  |  |  |  |  |  |  |  |  |
| Actual area of footing = |  |  | 3.3489 |  | m² |  |  |  |  |  |  |  | OK |  |
| Net Pressure acting upwads = |  |  | 100.81 |  | KN/m² |  |  |  |  |  |  |  |  |  |
| B. M. at the face of column about an axis = |  |  |  |  | 50.51 |  | KN-m |  |  |  |  |  |  |  |
| The effective depth required is given by |  |  |  |  |  |  |  |  |  |  |
|  |  |  | BM= 0.138 fck*bd^2 |  |  |  |  |  |  |  |
|  |  | d= | 100.00 | mm |  |  |  |  |  |  |
| Adopt d = 2 to 3 Times of calculated value of d for Shear considerations. |  |  |  |  |  |  |  |  |  |  |
|  |  | Adop. d= | 300 | mm |  |  |  |  |  |  |
|  |  | D= | 350 | mm |  |  |  |  |  |  |
| Check for one-way Shear action |  |  |  |  |  |  |  |  |  |  |
| The critical section is taken at distance d away from the face of column |  |  |  |  |  |  |  |  |  |  |
| Shear force Vu= |  | 81.17 | KN |  |  |  |  |  |  |  |
| Nominal shear stress (Tv) = |  |  | Vu/b*d = | 0.15 |  | N/mm² |  |  |  |  |
| Allowable Shear Stress of Concrete for pt% of Steel < = 0.15 & |  |  |  |  |  | ( M 20 ) |  | 0.28 |  |  |
| For 0.165% | Tc = | 0.292 | N/sq.mm | (From IS 456: 2000, table 19) |  |  |  |  |  |  |
|  |  | Hence, | OK |  |  |  |  |  |  |  |
|  | Tc should be equal to or greater than Tv |  |  |  |  |  |  |  |  |  |
| Check for two-way Shear action |  |  |  |  |  |  |  |  |  |  |
| The critical section is taken at a distance of 0.5d away from the face of column , |  |  |  |  |  |  |  |  |  |  |
|  | Shear force Vu= |  | 295.02 | KN |  |  |  |  |  |  |
| Nominal Shear Stress(Tv) = Vu/b'd (Where b' is the periphery of critical section ) |  |  |  |  |  |  |  |  |  |  |
|  | and | b' = | 2600 | mm |  |  |  |  |  |  |
| So, Nominal Shear Stress(Tv) = |  |  | 0.38 | N/mm² |  |  |  |  |  |  |
| Shear Strength of concrete is |  |  |  |  |  |  |  |  |  |  |
| Tc = | 1.12 | N/sq.mm |  |  |  |  |  |  |  |  |
| T'c = | Ks*Tc | Where Ks=(0.5+Bc) |  |  |  |  |  |  |  |  |
|  |  | Bc=length of shorter side of column/length of longer side of column |  |  |  |  |  |  |  |  |
|  |  | Bc= | 1 |  |  |  |  |  |  |  |
|  |  | Ks= | 1.5 | should not be greater than 1 |  |  |  |  |  |  |
|  |  | Take(Ks)= | 1 |  |  |  |  |  |  |  |
|  |  | T'c= | 1.12 | N/mm² |  |  |  |  |  |  |
|  | Hence, |  | OK | T'c should be greater than Tv |  |  |  |  |  |  |
| Area of Steel Calculation, |  |  |  |  |  |  |  |  |  |  |
| Area of Steel is given by formula, BM = 0.87* fy* At*{d-fy*At/(fck*b)} |  |  |  |  |  |  |  |  |  |  |
| Then, |  |  |  |  |  |  |  |  |  |  |
|  | Ast = | 394.14 | sq.mm | > Ast min |  | = 0.12% of b * d |  |  |  |  |
|  | Ast per m = | 215.38 |  |  |  | 360 |  | mm² |  |  |
|  | Req. Area of steel = |  | 360.00 | mm² |  |  |  |  |  |  |
|  | % of steel= |  | 0.072 |  |  |  |  |  |  |  |
| Steel requied as |  | 12 | mm bar @ | 314.16 |  | mm c/c distance. |  |  |  |  |
| Provide Steel as = |  | 12 | mm bar @ | 150 |  | mm c/c distance. |  |  |  |  |
|  | Provided steel = |  | 753.98 | mm² |  |  |  |  |  |  |
|  |  |  |  | Hence Ok. |  |  |  |  |  |  |

| Node No. | Footing | Footing Size | Rebar | Thickness |
| --- | --- | --- | --- | --- |
| A1, D1, A3, D3 | F1 | 5’0”X5’0” | Φ12@ 150mm c/c both direction | 1’-10” |
| C1, A2, D2 | F2 | 6’0”X6’0” | Φ12@ 150mm c/c both direction | 1’-10” |
| C2 & C3, B2 & C2 | CF | 11’-2”X7’0” | Φ12@ 150mm c/c both direction | 1’-10” |

The output of the design of footing is presented below. The construction shall follow the detail drawing of footing.

## ACCEPTABILITY CRITERIA

## 13.1   STORY DRIFT

Inter-storeyDrift(Load case:ULSEQx)

Inter-storeyDrift(Load case:ULSEQy)

Inter-storeyDrift(Load case:SLSEQx)

Inter-storeyDrift(Load case:SLSEQy)

To control overall deflection, the criteria given in clause 5.6.1 of NBC 105:2020 is applied. The maximum design horizontal deflection in any story due to the design lateral force shall be determined by multiplying the horizontal deflection found from Equivalent Static Method or Modal Response Spectrum Method by the Ductility factor (Rμ). For ultimate limit state and for serviceability limit state it shall be taken as equal to the horizontal deflections calculated either by Equivalent Static Method or Modal Response Spectrum Methods.

Furthermore, the drift shall not exceed maximum drift in the model. The maximum drift for ultimate limit state is 0.005203*4=0.0208 which is less than 0.025. The maximum drift for serviceability limit state is 0.004993 which is less than 0.006.

## DESIGN INPUT IN MUNICIPALITY FORMAT

- Geometrical Configuration ofStructure:

  - No. of Blocks:-1

  - No. of Storey:-2.5

  - Total Height of the Structure:-9.433m

- Structural Analysis andDesign:

  - Structural Analysis Software:- ETABS2020

  - Structural System: - RC buildings with Special Moment Resisting Frame SMRF.

  - Foundation System:-Isolated, Combined  and Strap Footing

  - Loading Parameters:

    - Dead Load:- As per IS875:1987 PartI

    - Live Load:- As per IS875:1987 PartII

    - Seismic Load (As perNBC 105:2020):-

  - Maximum Deflection of theBuilding:-

  - Maximum Drift in theBuilding:-

  - Load Combination Considered:-NBC 105:2020

  - Concrete Design code Referred:-  IS456:2000

  - Ductile Detailing codeReferred:-

IS13920:2016

  - Concrete Grade Used: Beam –M20

Slab – M20

Column – M25

Foundation – M20

  - Reinforcement Grade Used: -Fe500