STRUCTURAL ANALYSIS REPORT

FOR

RESIDENTIAL BUILDING OF

SABITA TIWARI

_______________

SEPTEMBER 2025

SUBMITTED TO:

LALITPUR METROPOLITON CITY

Designed By:

Sakriya Awal

NEC  NO.: 17303 Civil “A”

Table of Contents

LIST OF FIGURES	II

LIST OF TABLES	II

A	Features of the Building	1

A.1	General Information of the Project	1

A.2	Element sizes	1

A.3	Structural System and Foundation	1

B	Modelling of the Building	1

B.1	Material Properties (Concrete, Re-bar, Structural Steel)	1

B.2	Stiffness Modifiers	2

B.3	Slab and Diaphragm	2

B.4	Accidental Eccentricity	2

B.5	Support Condition	2

B.6	Structural Analysis Software Used	2

C	Adequacy of Actions to the Buildings	2

C.1	Load Patterns considered	2

C.2	Load Cases Considered	4

C.3	Mass Source Considered for Seismic Weight	4

C.4	Periods of vibration	4

C.4.1	Empirical Equations	4

C.4.2	Amplification of Approximate Period	4

C.5	Seismic Load Calculation Parameters	4

C.5.1	Base Shear Calculation from Equivalent Static Method	4

C.6	Load Combinations	6

D	Analysis of Building	6

D.1	Modal Analysis Results	6

D.2	Inter-Story Deflections	6

E	STRUCTURAL IRREGULARITY	9

E.1	Vertical Irregularity	9

E.2	Plan Irregularity	9

E.3	Conclusion	9

F	Adequacy of Member Design	10

F.1	Check for All Members Passed (for Element Design through Software)	10

F.2	Check for Column-Beam (C/B) Capacity Ratio	10

F.3	Design of Slabs	11

F.4	Design of Beams	12

F.5	Design of Columns	14

F.6	Check for Ductile Detailing	16

F.7	Design of Foundations	16

F.8	Design of Staircases	19

# LIST OF FIGURES

Figure 1 Storey Drift: Eqx ULS	7

Figure 2 Storey Drift: Eqy ULS	7

Figure 3 Storey Drift: Eqx SLS	8

Figure 4 Storey Drift: Eqy SLS	8

Figure 5 RCC Design Check	10

Figure 6 Column - Beam Capacity Ratio	10

Figure 7 RCC Design Results: Beam Longitudinal Rebar Ground Floor	12

Figure 8 RCC Design Results: Beam Longitudinal Rebar First Floor	13

Figure 9 RCC Design Results: Beam Longitudinal Rebar Second Floor	13

Figure 10 RCC Design Results: Beam Longitudinal Rebar Third Floor	14

Figure 14 RCC Design Results: Column Longitudinal Rebar Grid A	14

Figure 15 RCC Design Results: Column Longitudinal Rebar Grid B	15

Figure 16 RCC Design Results: Column Longitudinal Rebar Grid C	15

Figure 18 Footing Settlement	16

Figure 19 Soil Pressure on Footings	16

Figure 20 Design Results: Foundation Tie Beams Longitudinal Rebars	17

Figure 21  Design Results: Footing Top Bar	17

Figure 22 Design Results: Footing Bottom Bars	18

# LIST OF TABLES

Table 1 General Information of Building	1

Table 2 General Information on Structural Information of the Building	1

Table 3 Structural System and Foundations	1

Table 4 Effective Stiffness of Different Components	2

Table 5 Load Assessment for structural Analysis	3

Table 6 Live load intensity of Different Room	3

Table 7 Mass Sources for Seismic Weight	4

Table 8 Time period and Modal Mass Participation ratio	6

# Features of the Building

## General Information of the Project

Design Details:

The building has three floors made of reinforced concrete frames and filled with brick walls.

Table 1 General Information of Building

| 1 | Location of Building |  |
| --- | --- | --- |
| 2 | Number of Storeys Proposed for Building | G+1 |
| 3 | Number of Storeys Considered in Design | G+1 |
| 4 | Height of Typical Floor | 3.2 meters |
| 5 | Total Height of the Building super structure | 9.6 meters |
| 8 | Height considered for Fundamental Time Period | 6.4 meters |
| 9 | Occupancy Type of the Building according to the Building Code | Ordinary Structure Residential |

## Element sizes

Table 2 General Information on Structural Information of the Building

| Element | Description | Grade of Concrete | Remarks |
| --- | --- | --- | --- |
| Column | 350 mm x 350 mm | M20 | C1,C2 |
| Plinth Beam | 250 mm x 300 mm | M20 |  |
| Floor Beams | 350 mm x 450 mm | M20 |  |
|  | 300 mm x 400 mm | M20 |  |
| Slab | 125 mm | M20 |  |
| Foundation | Isolated footing, Combined Footing | M20 | SBC is taken as 120 kN/m2 |

## Structural System and Foundation

The chosen structural system for the building is a reinforced concrete (RC) building with a Special Moment Resisting Frame (SMRF). The columns and beams are carefully laid out in coordination with the architectural and services planning. This arrangement ensures that the structure can effectively support and transmit forces from earthquake motions, gravity, and live loads to the ground. The significance of this system increases as the building height increases. Hence, the key requirements for the structural systems are sufficient strength to prevent failure, adequate lateral stiffness, and efficient performance over the building's lifespan. The selection and arrangement of major structural elements to efficiently resist gravity and horizontal loads are essential in determining the building's structural form. Several factors influence the choice of structural form, including internal planning, construction materials and methods, external architectural treatment, placement and routing of service systems, magnitude and nature of horizontal loading, as well as the building's height and proportions.

All loads from the building's superstructure are transferred to the ground. If the foundation design is inadequate, all efforts put into designing the superstructure would be futile. Therefore, meticulous care must be taken in the design of foundations. The foundation design process begins with detailed field and soil investigation. Understanding the soil's geotechnical properties, including soil chemistry, is crucial to ensure the foundation performs reliably.

Table 3 Structural System and Foundations

| 1 | Type of Structural System | Moment Resisting Frames |
| --- | --- | --- |
| 2 | Type of Foundation | Isolated footing, Combined Footing |
| 3 | Type of Slab used | Conventional |

# Modelling of the Building

## Material Properties (Concrete, Re-bar, Structural Steel)

CONCRETE

According to NBC105:2025, The material should be taken as per Cl.2.1.

Minimum grade of structural concrete shall be M20, but M25 for buildings more than 12 m in height.

As the building is less than 12 m in height, M20 concrete has been taken for all the structural components.

STEEL

Steel reinforcement used shall be of,

a) Grade Fe 415 or less; or

b) High strength deformed steel bars produced by thermo-mechanical treatment process having elongation capacity of more than 15 percent; e.g. Grade Fe 500 and Fe 550.

Steel Grade of Fe500 has been used in the longitudinal and transverse reinforcement

## Stiffness Modifiers

Table 4 Effective Stiffness of Different Components

| No. | Component | Flexural Stiffness |  |
| --- | --- | --- | --- |
|  |  | Ultimate | Serviceability |
| 1 | Beam | 0.35 Ec Ig | 0.70 Ec Ig |
| 2 | Columns | 0.70 Ec Ig | 0.90 Ec Ig |
| 3 | Wall-Cracked | 0.50 Ec Ig | 0.70 Ec Ig |
| 4 | Wall-uncracked | 0.80 Ec Ig | 0.90 Ec Ig |

## Slab and Diaphragm

The column exhibits high rigidity in the vertical direction, while the floor system is assumed to have significantly higher stiffness in resisting horizontal loads, primarily due to the presence of the floor slab. The floor slab contributes to the overall in- plane rigidity, causing the columns, walls, and braces connected to that plane to behave as a cohesive unit when subjected to lateral forces. This system is commonly referred to as a rigid floor diaphragm, where the beam is seamlessly connected to the slab, resulting in minimal bending in the vertical plane. Conventional slab with rigid diaphragm has been assumed

## Accidental Eccentricity

Due to the eccentricity arising between the centre of mass and the centre of rigidity at each floor level of the building, code requires increase in shear forces on the lateral force resisting elements caused by the twisting about the vertical axis of the building. Hence design codes prescribe that the actual centre of mass shall be shifted by a stipulated value to consider accidental torsional force in the lateral force resisting elements, i.e. columns and shear walls. In this part of the checklist, accidental eccentricity to be considered in x-direction as per the clause 5.7 of NBC 105:2025 is to be listed. As per the clause, the value of accidental eccentricity shall

be ±0.05*b where b is the floor lateral dimension of the building perpendicular to the direction of lateral force EQx. Thus the design eccentricity at floor level i, edi,y will be equal to e +0.10*b where e is the actual static eccentricity in y-direction.

## Support Condition

If the building is supported by a pile or mat foundation, or an isolated footing on firm soil, the support condition is considered to be fixed. However, if the soil is less firm and the foundation type is isolated, the support condition will be closer to a hinge support. Therefore, it is necessary for the designer to specify the appropriate type of support used in the building model in this section of the checklist. For this design, the foundation is assumed to be fixed.

## Structural Analysis Software Used

The building was modelled using the ETABS 22.7.0 software. ETABS 22.7.0 is a specialized program designed specifically for the analysis and design of building systems. It offers a user-friendly interface and advanced features, making it both sophisticated and easy to use. The software provides intuitive modelling capabilities, powerful analytical tools, efficient design procedures, robust numerical methods, and supports various international design codes. With its ability to handle complex structures and nonlinear behaviours, ETABS 22.7.0 is widely preferred by structural engineers in the building industry. However, finite element software like SAP2000 Staad.Pro, RISA3D, etc. are also present.

For the design of foundation,  SAFE, is  used.

# Adequacy of Actions to the Buildings

## Load Patterns considered

- Dead

- Wall Load

- Lift Load

- Floor Finish

- EQx ULS

- EQx SLS

- EQy ULS

- EQy SLS

- Live 1

- Live 2

Load calculations are performed using the NBC 102:1994 as a reference. The first step is to select the type of material and determine the unit weight values from the aforementioned code. The thickness of the material is chosen based on the design requirements. With the knowledge of the area, thickness, and unit weight of the materials, the loads on each section are determined.

The brick masonry is constructed with careful adherence to standard practices, including pre-soaking the bricks in water, ensuring level bedding with full mortar coverage, breaking vertical joints from course to course, and filling them with mortar completely.

Bricks: Standard rectangular bricks are used, which are well burnt, hand-formed or machine-made, and possess a crushing strength of at least 3.5 N/mm². The preferred brick size is 230 x 115 x 57 mm, with 10 mm thick horizontal and vertical mortar joints.

Tolerances: For thick walls, acceptable tolerances are -10 mm on length, -5 mm on width, and ±3 mm on thickness.

Wall Thickness: The minimum thickness is half-brick, while the maximum thickness is one brick.

Mortar: Cement-sand mixes of 1:6 and 1:4 are adopted for one-brick and half-brick thick walls, respectively. Adding a small amount of freshly hydrated lime, in a ratio of ¼ to ½ of the cement, enhances the mortar's plasticity without compromising its strength. Therefore, adding lime within these limits is encouraged.

Plaster: All plasters should have a cement-sand mix with a minimum ratio of 1:6 and a cube crushing strength of at least 3 N/mm² after 28 days.

Wall Thicknesses: Internal walls have a thickness of up to 115 mm for brick walls or equivalent materials, while external walls have a thickness of up to 230 mm for brick walls or equivalent materials.

Table 5 Load Assessment for structural Analysis

| 1 | Unit Weights of materials |  |  |
| --- | --- | --- | --- |
|  | Brick Masonry | 19.5 | kN/m3 |
|  | Screed | 21 | kN/m3 |
|  | Reinforced Concrete | 25 | kN/m3 |
|  | Cement Plaster | 20.4 | kN/m3 |
|  | Marble | 26.7 | kN/m3 |
| 2 | Height of Beams, walls and parapet Walls |  |  |
|  | Height of Parapet Wall | 0.9 | m |
|  | Wall Thickness | 0.23 | m |
|  | Plaster Thickness | 0.02 | m |
| 3 | Dead Loads of Walls |  |  |
|  | Dead load of 230mm thick wall with 10% opening | 12.11 | kN/m |
|  | Dead load of 230mm thick wall with 30% -40% opening | 9 | kN/m |
|  | Dead load of 115mm thick wall with 10% opening | 6.5 | kN/m |
|  | Dead load of Parapet wall | 5 | kN/m |

The live loads for the floor and roof are determined using IS 875 part 2, as referenced by the National Building Code. For residential buildings, the appropriate live load values specified in Table 1 of IS 875 Part 2 are considered. Since the building is intended for residential use, the loading intensity for dwelling houses under the

residential category is taken into account.

Table 6 Live load intensity of Different Room

| S.N. | Occupancy Classification | Uniformly Distributed Load |
| --- | --- | --- |
|  | Dwelling Houses |  |
| 1 | All rooms and kitchens | 2.0 kN/m2 |
| 2 | Toilet and bath rooms | 2.0 kN/m2 |
| 3 | Corridors, passages, staircases | 3.0 kN/m2 |
| 4 | Balconies | 3.0 kN/m2 |
| 5 | Flat sloping or curved roof with slopes up to and including 10 degrees |  |
|  | Access provided | 1.5 kN/m2 |
|  | Access not provided except for maintenances | 0.75 kN/m2 |

## Load Cases Considered

- Dead

- Live

- Modal

- EQx ULS

- EQx SLS

- EQy ULS

- EQy SLS

## Mass Source Considered for Seismic Weight

Table 7 Mass Sources for Seismic Weight

| Load | Multiplier |
| --- | --- |
| Live<3 kN/m2 | 0.25 |
| Live>=3 kN/m2 | 0.5 |
| Dead Loads ( Self weight, walls, floor finishes etc) | 1 |

## Periods of vibration

The fundamental time period shall be determined using following methods:

### Empirical Equations

T1=ktH34

Here,

kt = 0.075

H = Height of the building from foundation or from top of a rigid basement = 6.4 m

T1 = 0.301784023 s

### Amplification of Approximate Period

The approximate fundamental time period calculated using empirical equation in section 5 1.2 shall be increased by a factor of 1.25.

Amplified Approximate Fundamental time period =0.377230029 s

## Seismic Load Calculation Parameters

### Base Shear Calculation from Equivalent Static Method

Elastic Site Spectra for horizontal loading is given by C(t)=Ch(T)*Z*I as per cl 4.1.1

Where,

Ch(t)=Spectral Shape factor as per cl.4.1.2

Z= Seismic Zoning Factor as per 4.1.4

I=Importance factor as per 4.1.5

#### Spectral Shape Factor, Ch (T)

| Fundamental Time Period = | 0.377230029 s |  |
| --- | --- | --- |
| Site Soil Category = | Very Soft Soil |  |
| Ta = | 0.5 |  |
| Tc = | 2 |  |
| Soil Type = | D |  |
| α = | 2.25 |  |
| k = | 0.8 |  |
| 𝐶ℎ(𝑇)= | 1+(α-1)x(T/Ta) | if T<Ta |  |
|  | α | if Ta≤T≤Tc |  |
|  | α[K+(1-K)(Tc/ T)2](Tc/ T)2 | if Tc≤T≤6 |  |

Spectral Shape Factor Ch(T)= 1.943075072

#### Seismic Zoning Factor (Z)

| Location= | Kathmandu |
| --- | --- |
| PGA (Z)= | 0.35 |
| Structural Types = | Ordinary Structures |
| Importance Factor = | 1 |

Elastic Site Spectra for Horizontal loading C(T) =Ch(T)*Z*I =0.680076275

Elastic Site Spectra for Serviceability limit state, Cs(T) = 0.20 C (T) =0.136015255

#### Horizontal base shear coefficient

Ultimate Limit State CdT1=C(T1)Rμ×Ωu

Where,

C (T1) = Elastic Site Spectra as per 4.1.1

Rμ = Ductility Factor as per 5.3

𝛀u = Over strength Factor for ULS as per 5.4

Serviceability Limit State CdT1=C(T1)Ωs

Where,

Cs(T1) = Elastic Site Spectra determined for Serviceability Limit State as per 4.2

𝛀s = Over strength Factor for SLS as per 5.4

| C (T1) = | 0.680076275 |
| --- | --- |
| Cs(T1)= | 0.136015255 |
| Structural System = | Reinforced Concrete Moment Resisting Frame |
| Ductility Factor Rμ = | 4 |
| Over strength Factor Ultimate limit State Ωu = | 1.5 |
| Over strength Factor Serviceability limit State Ωs= | 1.25 |
| Cd(𝑇1)=C(𝑇1)/Rμ x Ωu | 0.113346046 |
| Cd(T1)=Cs(T1)/ Ωs | 0.108812204 |

#### Horizontal seismic base shear

V = Cd (T1) W

Where,

Cd (T1) = Horizontal base shear coefficient as per 6.1

W = Seismic Weight of the structure as per 5.2

#### Vertical distribution of seismic forces

Fi=WihikinWihik×V

Where,

Wi = seismic weight of the structure assigned to level ‘i’;

hi= height (m) from the base to level ‘i’;

n= total number of floors/levels

V= horizontal seismic base shear calculated as per 6.2

k= an exponent related to the structural period

for structure having time period T≤0.5sec, k=1

for structure having time period T≥2.5sec, k=2

for structure having period between 0.5 sec and 2.5 sec, k shall be determined by linear interpolation between 1 and 2.

k = 1

## Load Combinations

When lateral load resisting elements are oriented along mutually orthogonal horizontal directions, the structure system is considered to be non-parallel and load combinations as specified in clause 3.6.2 of NBC 105:2025 is used for design.

| - 1.2DL + 1.5LL - 0.9 DL +Ex - 0.9 DL -Ex - 0.9 DL +Ey - 0.9 DL -Ey | - DL+0.3 LL +Ex - DL+0.3 LL -Ex - DL+0.3 LL +Ey - DL+0.3 LL -Ey |
| --- | --- |

# Analysis of Building

## Modal Analysis Results

NBC105:2025 clause 7.3 states that number of modes to be used in the analysis should be such that the sum total of modal masses of all modes considered is at least 90 percent of the total seismic mass of the structure. The analysis was carried out for 12 modes so that the mass participation satisfies this criterion in both orthogonal directions. The table shows time period and mass participation ratio for all modes.

Table 8 Time period and Modal Mass Participation ratio

| Case | Mode | Period(Sec) | UX | UY | SumUX | SumUY |
| --- | --- | --- | --- | --- | --- | --- |
| Modal | 1 | 0.627 | 0.0189 | 0.8056 | 0.0189 | 0.8056 |
| Modal | 2 | 0.617 | 0.7868 | 0.0205 | 0.8057 | 0.8262 |
| Modal | 3 | 0.575 | 0.0004 | 0.0148 | 0.8062 | 0.8409 |
| Modal | 4 | 0.312 | 0.0033 | 0.0144 | 0.8095 | 0.8553 |
| Modal | 5 | 0.311 | 0.1253 | 0.0003 | 0.9348 | 0.8556 |
| Modal | 6 | 0.291 | 0.00004 | 0.0855 | 0.9348 | 0.9411 |
| Modal | 7 | 0.199 | 0.0005 | 0.0289 | 0.9353 | 0.97 |
| Modal | 8 | 0.176 | 0.0537 | 0.0021 | 0.989 | 0.9721 |
| Modal | 9 | 0.164 | 0.0014 | 0.0192 | 0.9904 | 0.9913 |
| Modal | 10 | 0.03 | 6.936E-07 | 0.000007755 | 0.9904 | 0.9913 |
| Modal | 11 | 0.029 | 0 | 0.00001774 | 0.9904 | 0.9913 |
| Modal | 12 | 0.028 | 0 | 0.0028 | 0.9904 | 0.9942 |

## Inter-Story Deflections

The ratio of the inter-story deflection to the corresponding story height shall not exceed:

0.025 at ultimate limit state

0.006 at serviceability limit state

Figure 1 Storey Drift: Eqx ULS

Design horizontal Inter-Story Drift = 0.001763 x 3.2 = 0.0056< 0.025, OK

Figure 2 Storey Drift: Eqy ULS

Design horizontal Inter-Story Drift = 0.002151 x 3.2 = 0.0068 < 0.025, OK

Figure 3 Storey Drift: Eqx SLS

Design horizontal Inter-Story Drift = 0.001693 < 0.006, OK (For SLS)

Figure 4 Storey Drift: Eqy SLS

Design horizontal Inter-Story Drift = 0.002065 < 0.006, OK (For SLS)

# STRUCTURAL IRREGULARITY

One of the basic virtues of earthquake resistant design of structures is that the structure shall be regular as much as possible. It is observed that the structures with simple and regular configurations suffer much less damage during a large earthquake in comparison to the structures with irregular configurations. At the planning stage itself, the Designer should try to make the structure as regular as possible. Clause 5.4 of NBC 105:2025 has given provisions to check the structural irregularity of structures.

## Vertical Irregularity

- Weak Storey: A storey is considered weak if the lateral strength is less than 80% of that of the storey above — Not observed.

- Soft Storey: A storey is soft if its stiffness is less than 70% of the storey above or less than 80% of the average stiffness of the three storeys above — Not observed.

- Vertical Geometric Irregularity: Occurs when the horizontal dimension of the lateral force-resisting system in any storey exceeds 130% of that in an adjacent storey — Not observed.

- In-plane Discontinuity: Exists when vertical lateral force-resisting elements are offset in plane — Not observed.

- Mass Irregularity: Occurs if the effective mass differs by more than 50% from the adjacent storey — Not observed.

## Plan Irregularity

- Torsional Irregularity: When maximum displacement at one end exceeds 1.5× the minimum at the other end — Not observed.

- Extreme Torsional Irregularity: Maximum displacement exceeds 2.5× minimum (not permitted) — Not observed.

- Re-entrant Corners: Projection exceeds 15% of overall dimension — Not observed.

- Diaphragm Discontinuity: Openings >50% or stiffness variation >50% between storeys — Not observed.

- Out-of-plane Offset: Discontinuity in vertical lateral force-resisting elements — Not observed.

## Conclusion

No vertical or plan irregularities were identified. The building is regular and complies with NBC 105:2025 for earthquake-resistant design.

# Adequacy of Member Design

## Check for All Members Passed (for Element Design through Software)

Figure 5 RCC Design Check

## Check for Column-Beam (C/B) Capacity Ratio

Figure 6 Column - Beam Capacity Ratio

To prevent the progressive collapse of a structure caused by the failure of columns at lower levels, the design follows the principle of Strong-Column-Weak Beam Design. This design approach ensures that the columns are designed to be stronger than the beams. This helps the structure effectively absorb and dissipate seismic energy, reducing the risk of complete collapse. By allowing plastic hinges to form in the beams, the structure gains increased ductility, enabling it to undergo significant lateral displacements while maintaining its integrity. This design strategy enhances the structure's ability to withstand seismic forces and mitigate potential damage.

## Design of Slabs

Slab are designed as per IS 456 code.

| Concrete |  |  |
| --- | --- | --- |
| Grade | M-20 |  |
| fck(N/mm2) | 20 |  |
| Steel |  |  |
| fu(N/mm2) | 500 |  |
| Lx | 5.3 |  |
| Ly | 5.3 |  |
| Ly/Lx | 1 |  |
| slab type | two adjecent edge discontinous |  |
| BM coef | short span | long span |
| negative at continious | 0.046 | 0.047 |
| positive at mid | 0.035 | 0.035 |
| Load Calculation |  |  |
| Slab depth | 0.125 |  |
| effective depth | 0.087 |  |
| load on slab |  |  |
| Self weight of slab(kN/m2) | 3.125 |  |
| Floor finish(kN/m2) | 1.500 |  |
| Partition wall loads (kN/m2) | 2.000 |  |
| Live load(kN/m2) | 2.000 |  |
| total | 8.625 |  |
| Factored load | 12.938 |  |
| BM Calculation |  |  |
| lx | 5.3 |  |
| ly | 5.3 |  |
| w | 12.938 |  |
|  | Top | Bottom |
| Mx | 16.717 | 12.720 |
| My | 17.080 | 12.720 |
| For bottom bar x dir |  |  |
| d | 0.087 |  |
| Ast required | 336.127 |  |
| Provide bar diameter of | 10.000 |  |
| spacing | 150.000 |  |
| Ast provided | 523.599 | ok |
| For bottom bar y dir |  |  |
| d | 0.079 |  |
| Ast required | 370.173 |  |
| Provide bar diameter of | 10.000 |  |
| spacing | 150.000 |  |
| Ast provided | 523.599 | ok |
| For top bar x dir |  |  |
| d= | 0.087 |  |
| Ast required | 441.781 |  |
| Provide bar diameter of | 10.000 |  |
| spacing | 150.000 |  |
| Ast provided | 523.599 | ok |
| For top bar y dir |  |  |
| d | 0.079 |  |
| Ast required | 497.110 |  |
| Provide bar diameter of | 10.000 |  |
| spacing | 150.000 |  |
| Ast provided | 523.599 | ok |

## Design of Beams

Beam Design is done by Etabs using the IS 456 code.

Figure 7 RCC Design Results: Beam Longitudinal Rebar Ground Floor

Figure 8 RCC Design Results: Beam Longitudinal Rebar First Floor

Figure 9 RCC Design Results: Beam Longitudinal Rebar Second Floor

Figure 10 RCC Design Results: Beam Longitudinal Rebar Third Floor

## Design of Columns

Column Design is done by Etabs using the IS 456 code.

Figure 14 RCC Design Results: Column Longitudinal Rebar Grid A

Figure 15 RCC Design Results: Column Longitudinal Rebar Grid B

Figure 16 RCC Design Results: Column Longitudinal Rebar Grid C

## Check for Ductile Detailing

Refer to the structural drawing to verify whether proper ductile detailing has been done in the beam, column, joints etc. Verify the other requirements and also the ductile detailing requirements of the building as per Annex A of the code

## Design of Foundations

The foundations are designed using SAFE 22.3.0.

SBC taken 120 kN/m2, K= 14400

Maximum displacement =15.6mm < 25 mm ok

Figure 18 Footing Settlement

Figure 19 Soil Pressure on Footings

Figure 20 Design Results: Foundation Tie Beams Longitudinal Rebars

Figure 21  Design Results: Footing Top Bar

Figure 22 Design Results: Footing Bottom Bars

Footing Slab thickness= 400mm

| Footing at Grid | Type | Size | Depth | Bottom Rebar | Top Rebar |
| --- | --- | --- | --- | --- | --- |
| A1 | Isolated | 1700x1700 | 400 | 12 dia at 150mm spacing bothways |  |
| A3,A4 | Combined | 2300x5100 | 400 | 12 dia at 150mm spacing bothways | 8 dia at 150mm spacing bothways |
| A5 | Isolated | 1700x1700 | 400 | 12 dia at 150mm spacing bothways |  |
| B1 | Isolated | 2500x2500 | 400 | 12 dia at 150mm spacing bothways |  |
| B3,B4 | Combined | 3100x5300 | 400 | 12 dia at 125mm spacing bothways | 8 dia at 150mm spacing bothways |
| B5 | Isolated | 2700x2700 | 400 | 12 dia at 150mm spacing bothways |  |
| C1 | Isolated | 1700x1700 | 400 | 12 dia at 150mm spacing bothways |  |
| C2 | Isolated | 2500x2500 | 400 | 12 dia at 150mm spacing bothways |  |
| C4 | Isolated | 2500x2500 | 400 | 12 dia at 150mm spacing bothways |  |
| C5 | Isolated | 1900x1900 | 400 | 12 dia at 150mm spacing bothways |  |

## Design of Staircases

Staircase is designed as per IS 456 slab design.

| Concrete |  |  |
| --- | --- | --- |
| Grade | M-20 |  |
| fck(N/mm2) | 20 |  |
| Steel |  |  |
| fu(N/mm2) | 500 |  |
| Width of Flight | 1.200 |  |
| Landing Width | left | Right |
|  | 1.2 | 1.2 |
| Riser | 0.166 |  |
| Tread | 0.279 |  |
| No of Treads | 10 |  |
| Support Beam Width | 0.250 |  |
| Load Calculation |  |  |
| Effective span | 5.19 |  |
| width of flight | 2.79 |  |
| Waist Slab depth | 0.150 |  |
| effective depth | 0.112 |  |
| sloping waist length for one tread | 0.325 |  |
| load on flight |  |  |
| Self weight of waist slab(kN/m2) | 4.364 |  |
| Self weight of steps(kN/m2) | 2.075 |  |
| Floor finish(kN/m2) | 1.500 |  |
| Live load(kN/m2) | 3.000 |  |
| total | 10.939 |  |
| load on landing |  |  |
| Self weight of slab(kN/m2) | 3.750 |  |
| Floor finish(kN/m2) | 1.500 |  |
| Live load(kN/m2) | 3.000 |  |
| total | 8.250 |  |
| BM Calculation |  |  |
| R1 | 25.159 |  |
| R2 | 25.159 |  |
| point of max BM | 2.595 |  |
| Max BM | 34.895 |  |
| Ast required | 851.894 |  |
| Provide bar diameter of | 12.000 | mm |
| spacing | 150.000 | mm |
| Ast provided | 904.779 | ok |
| Distribution bar of diameter | 10.000 | mm |
| Ast required(mm2) | 180.000 |  |
| Spacing | 150.000 | mm |
| Ast provided | 523.599 |  |